- Altiplano NBI for TRUE Edition 0.1

No part of this publication may be reproduced, stored in a retrieval system, or transmitted in any form or by any means, electronic, mechanical, photocopying, recording or otherwise and it may not be disclosed to any third party without the prior written permission of Nokia. It is the responsibility of the user of this document to verify in the repository that it is the most current version. Any printed document version is considered an uncontrolled copy.

Document Approval

Name(s) of those who are approving this document for release.

| Nokia Approval |  | TRUE Approval |  |
| --- | --- | --- | --- |
| Role |  | Role |  |
| Name |  | Name |  |

Document History

Document history is entered in order of latest release first, oldest release last

| Edition | Date | Author | Details of Change |
| --- | --- | --- | --- |
| 0.1 | 2024-03-13 | Ronghuan Tu | First Draft. |

Documentation location

Location of template in the Nokia archive.  
Always download the latest version of the template from URL below.

Electronic Location:

Table of Contents

Altiplano NBI for TRUE	1

1	Introduction	6

2	Scope	7

3	Overview	8

4	Altiplano NBI interface	9

4.1	Authentication	9

4.1.1	Authentication for intent	9

4.2	Test & Diagnosis (IBN ACTION)	10

4.2.1	Get MAC address for CFM	10

4.2.2	Maintenance Association	11

4.2.3	Get ONT Detail Informaiton	12

4.2.4	Get UNI Detail Information	14

4.2.5	Reboot ONT	16

4.2.6	Lock UNI	16

4.2.7	Unlock UNI	17

4.2.8	Show fiber SFP inventory	17

4.2.9	Show Fiber Optics Measurement	19

4.2.10	Get VLAN Sub-Interface Information	22

4.2.11	Query VLAN Association DHCP Sessions	23

4.2.12	Query All MF OLT	25

4.2.13	Query All Fiber under OLT	30

4.2.14	Show Equipment Slot inventory	32

4.2.15	Enable/Disable PON Utilization Monitoring	45

4.2.16	Show FDB Mac Address	45

4.2.17	Show Uplink Port SFP Diagnostics and Inventory	47

4.2.18	Restart	49

4.2.19	Show Last Restart Time of Slot	50

4.2.20	Show  NT Protection State	51

4.2.21	Show Forwording Database MAC	53

4.2.22	Show  CFM MAC Per Board	55

4.2.23	Show Optics Inventory Information	56

4.2.24	Show LAG Statistics	58

4.3	RC-Proxy query	60

4.3.1	NC Browser	60

4.3.2	RC-Proxy query Url	61

4.3.3	ONT Admin status / Operational status / Last change date / Ranged / SN / Distance	62

4.3.4	ONT Software Version	63

4.3.5	ONT Optical Power	64

4.3.6	ONT Eth Card / Port State	65

4.3.7	ONT Eth Auto Detection Configuration	65

4.3.8	ONT Eth Port Actual Speed	66

4.3.9	VLAN Association Data	67

4.3.10	Bridge Port Vlan	68

4.3.11	NTD UNI-D Counters	68

4.3.12	VLAN Association Counters	69

4.3.13	PON Admin / Operational status	72

4.3.14	PON optical data	73

4.3.15	Query VLAN Association DHCP Sessions	74

4.4	PM interface	75

4.4.1	PM counter collection mode	75

4.4.2	Which counters to be collected	77

4.4.3	IPFIX related profile	78

4.4.4	PM collection configuration on AAC	93

4.4.5	Bulk collection NBI	94

4.5	Inventory interface	101

4.5.1	ES-search in general	101

4.5.2	Intent inventory	102

4.5.3	Device inventory	115

# Introduction

The goal of this documentation is to provide a low-level design of Nokia Altiplano integration with OSS

# Scope

This document will talk about for the Ligntspan OLT related OSS NBI interfaces, including performance and inventory parts.

# Overview

Altiplano, together with Lightspan OLT forms the core part of SDAN. Altiplano provides a serial of NBI for the OSS integration.

Restful and Kafka are common interfaces for easy OSS integration

# Altiplano NBI interface

Altiplano NBI includes several parts, as

- Authentication

- Test & Diagnosis

- RC-proxy device query

- Telemetry collection

- Inventory collection

## Authentication

There are two authentication endpoints. One is for intent and Elasticsearch API authentication, another one for T &D query.

### Authentication for intent

|  | Function | METHOD | Endpoint |
| --- | --- | --- | --- |
| 1 | get-token | POST | https://<server>/altiplano-sso/realms/master/protocol/openid-connect/token |

OSS send base64 encoded username/password to Altiplano to ask for the Token.

Headers

"Authorization": "Basic YWRtaW51c2VyOnBhc3N3b3Jk",

And OSS expects the response from Altiplano as:

{

"access_token": "eyJhbGciOiJSUzI1NiIsInR5cCIgOiAiSldUIiwia2lkIiA6ICJnWmlKZHZkWkkybFBwTHh3a2xPZDQteXBnUW5SdTFHV052LTFmLTR3QU44In0",

"expires_in": 1800,

"refresh_expires_in": 86400,

"refresh_token": "eyJhbGciOiJIUzI1NiIsInR5cCIgOiAiSldUIiwia2lkIiA6ICI4YTQ0Yzk5OC01MjY4LTQ3MDgtODI3NC1hNTQwNDYxZjRlY2EifQ",

"token_type": "Bearer",

"not-before-policy": 0,

"session_state": "f645e6d0-643a-490d-a59a-465be4cba7f5",

"scope": "email profile membership altiplano-audience"

}

Main Parameters

| Variable & Parameter | Value |
| --- | --- |
| <server> | Configured by helm install --name parameter when install Altiplano solution. |

## Test & Diagnosis (IBN ACTION)

### Get MAC address for CFM

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Query MAC Address | Birth Certificate Loopback PM Verification | POST | {{releasename}}-altiplano-ac/rest/restconf/data/ibn:ibn/intent={{ONT Name}}%23{{MD ID}},cfm/intent-specific-data/cfm-mep-address-action:cfm-show-mac |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{UNI Port}} | The pattern of {{UNI Port}} is like “LAN1” or “10GE” |

Sample request body:

{

"cfm-mep-address-action:input": {

"port-id": "LAN1"

}

}

Expected response:

{

"cfm-mep-address-action:output": {

"mac-address": "70:d9:31:6c:96:bc"

}

}

Screenshot for NC Browser GUI:

### Maintenance Association

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Query | Query Maintenance Association | Loopback | GET | nokia-altiplano-ac /rest/restconf/data/ibn:ibn/intent={{ONT Name}}%23{{MD Name}},cfm/intent-specific-data/cfm:cfm/ |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Expected response:

{

"cfm:cfm": {

"md-profile": "MD_Profile_Level2",

"maintenance-association": [

{

"char-String": "MA1",

"ma-profile": "Default_Maint_Association",

"ma-id": "MA-1",

"maintenance-endpoint": [

{

"mg-id": "MG-1",

"mep-id": "102",

"service": "NEBS#AVC123456789012",

"mep-profile": "Default_Up_MEP_Profile",

"mep-type": "vlan-sub-interface"

}

]

}

],

"char-Strings": "MD1",

"location": "ONT"

}

}

Screenshot for NC Browser GUI:

### Get ONT Detail Informaiton

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Get ONT Detail Informaiton | ONT Admin Status ONT Op. Status bponOntEquipId ONT Ranged ONT SN ONT SW version Distance Optical Power NTD last change date | POST | {{releasename}}-altiplano-ac/rest/restconf/data/ibn:ibn/intent={{ONT Name}},ont/intent-specific-data/ont-action:ont-show-detail |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{ONU Name}} | The ONU name configured in ont intent. |

Note: {{ONT Name}} is the ONT Intent target-name. It is NTD name as NTD

Expected response:

{

"ont-action:output": {

"ont-detail": {

"ds-history-time-stamp": "",

"us-current-in-bip-errors": "",

"ds-history-in-bip-errors": "",

"oper-status": "up",

"us-history-uncorrectable-fec-codewords": "",

"laser-temperature": "0",

"us-current-in-fec-codewords": "",

"rx-power-dbm-at-olt": "-24.2",

"passive-sw-version": "3FE54613AOCK32",

"us-current-uncorrectable-fec-codewords": "",

"onu-fiber-distance": "46",

"us-history-in-fec-codewords": "",

"us-history-in-bip-errors": "",

"model-name": "__________I-240G-R__",

"presence-status": "onu-present-and-on-intended-channel-termination",

"detected-serial-number": "ALCLF8B466FC",

"ds-history-in-fec-codewords": "",

"ds-current-uncorrectable-fec-codewords": "",

"ds-history-in-bip-protected-words": "",

"tx-power": "0",

"ds-current-in-bip-protected-words": "",

"tx-bias": "0",

"ds-current-in-fec-codewords": "",

"rx-power": "0",

"last-changed-date": "2024-01-04T02:21:59+00:00",

"us-current-in-bip-protected-words": "",

"rx-power-dbm": "0",

"ds-history-uncorrectable-fec-codewords": "",

"us-history-time-stamp": "",

"admin-status": "up",

"tx-power-dbm": "0",

"us-history-in-bip-protected-words": "",

"active-sw-version": "3FE54613AOCK36",

"onu-id": "0",

"ds-current-in-bip-errors": ""

}

}

}

Screenshot for NC Browser GUI:

### Get UNI Detail Information

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Get UNI Detail Information | Get UNI-D Status NTD UNI-D Counters Get ONT Ethernet Port Parameters bponLanEtherPortOpstatConfigIndicator bponLanEtherPortAutoDetectConfig | POST | {{releasename}}-altiplano-ac/rest/restconf/data/ibn:ibn/intent={{ONT Name}},ont/intent-specific-data/ont-uni-customized-action:uni-detail |

Headers:

| Key | Value |
| --- | --- |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{ONT Name}} | The ONT Name configured in ont intent. |
| {{UNI Port}} | The pattern of {{UNI Name}} is like “LAN1” or “10GE” Note: {{UNI Name}} is predefined in ONT-Template. For the 1G UNI port, it is LAN1, LAN2, LAN3 and LAN4. |

Sample request body:

{

"ont-uni-customized-action:input": {

"uni-id": "{{UNI Port}}"

}

}

Expected respond XML result:

{

"ont-uni-customized-action:output": {

"uni-detail": {

"config-speed": "eth-if-speed-1gb",

"last-change": "2023-09-19T02:39:00+00:00",

"oper-status": "up",

"duplex": "full",

"out-discards": "0",

"config-duplex": "full",

"speed": "1000",

"auto-negotiation": "enabled",

"out-octets": "96026150",

"in-octets": "15576928",

"admin-status": "up",

"phys-address": "00:00:00:00:00:00",

"in-discards": "0"

}

}

}

Screenshot for NC Browser GUI:

### Reboot ONT

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Reboot ONT with Active Image | NTD Reset | POST | {{releasename}}-altiplano-ac/rest/restconf/data/ibn:ibn/intent={{ONT Name}},ont/intent-specific-data/ont-action:reset-ont |

Headers:

| Key | Value |
| --- | --- |
| Authorization | JWT token |

Sample request body:

{

"ont-action:input": {

"reset": "active-image"

}

}

Expected respond XML result:

Respond 204 No Content

### Lock UNI

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Lock UNI | Reset UNI port | POST | {{releasename}}-altiplano-ac/rest/restconf/data/ibn:ibn/intent={{ONT Name}},ont/intent-specific-data/ont-uni-customized-action:lock-uni |

Headers:

| Key | Value |
| --- | --- |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{UNI Name }} | The pattern of {{UNI Name}} is like “LAN1” or “10GE” |

Sample request body:

{

"ont-uni-customized-action:input": {

"uni-id": "{{UNI Name }}"

}

}

Expected respond XML result:

Respond 204 No Content

### Unlock UNI

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Unlock UNI | Reset UNI port | POST | {{releasename}}-altiplano-ac/rest/restconf/data/ibn:ibn/intent={{ONT Name}},ont/intent-specific-data/ont-uni-customized-action:unlock-uni |

Headers:

| Key | Value |
| --- | --- |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{UNI Name}} | The pattern of {{UNI Name}} is like “LAN1” or “10GE” |

Sample request body:

{

"ont-uni-customized-action:input": {

"uni-id": "{{UNI Name }}"

}

}

Expected respond XML result:

Respond 204 No Content

### Show fiber SFP inventory

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Show Fiber SFP Inventory | Fiber SFP inventory | POST | {{protocol}}:///{{server}}/{{base-url}}/rest/restconf/data/ibn:ibn/intent={{Fiber Name}},fiber/intent-specific-data/pon-sfp:show-sfp-inventory |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{Fiber Name}} | The Fiber name configured in fiber intent. SWOLTxxxxxx.Lyy.Pzz​ e.g.​ SWOLT0000333.L13.P05 |

Expected response:

{

"pon-sfp:output": {

"pon-optics": [

{

"wavelength": 1490,

"tx-power": "5.2",

"admin-status": "unlocked",

"last-change": "2023-10-29T08:37:15+00:00",

"module-temperature": "33.1",

"tx-bias-current": "12.338",

"oper-status": "enabled",

"module-voltage": "3.353",

"name": "PON_1_GPON"

},

{

"wavelength": 1577,

"tx-power": "6.1",

"admin-status": "unlocked",

"last-change": "2023-10-25T00:51:56+00:00",

"module-temperature": "33.1",

"tx-bias-current": "52.568",

"oper-status": "enabled",

"module-voltage": "3.353",

"name": "PON_1_XGS"

}

],

"inventory": {

"oui": "-",

"hardware-revision": "10",

"supply-voltage": "3.353",

"serial-number": "SPXNCN22120MBL",

"mfg-name": "SUPERXON LTD.",

"oper-state": "enabled",

"mfg-date": "2022-03-26T00:00:00-00:00",

"admin-state": "unlocked",

"fiber-type": "single-mode",

"clei-code": "VBIUADWBAA",

"sfp-name": "PONSFP_1",

"part-number": "3FE47581BF",

"temperature": "33.1"

}

}

}

Screenshot for Altiplano GUI:

### Show Fiber Optics Measurement

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Show measurement | Fiber Optics measurement | POST | {{protocol}}:///{{server}}/{{base-url}}/rest/restconf/data/ibn:ibn/intent={{Fiber Name}},fiber/intent-specific-data/optics:show-optical-measurements |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{Fiber Name}} | The Fiber name configured in fiber intent. SWOLTxxxxxx.Lyy.Pzz​ e.g.​ SWOLT0000333.L13.P05 |

Expected response:

{

"optics:output": {

"pon-optics": [

{

"wavelength": 1490,

"tx-power": "5.2",

"admin-status": "unlocked",

"last-change": "2023-09-11T05:51:05+00:00",

"module-temperature": "33.3",

"tx-bias-current": "12.338",

"oper-status": "enabled",

"module-voltage": "3.353",

"name": "PON_1_GPON"

},

{

"wavelength": 1577,

"tx-power": "6.1",

"admin-status": "unlocked",

"last-change": "2023-09-11T05:51:05+00:00",

"module-temperature": "33.4",

"tx-bias-current": "52.568",

"oper-status": "enabled",

"module-voltage": "3.352",

"name": "PON_1_XGS"

}

],

"ont-optics": [

{

"tx-power": "3",

"module-temperature": "28",

"rx-power": "-19.1",

"last-changed-date": "2023-09-12T08:10:44+00:00",

"tx-bias-current": "12.908",

"oper-status": "up",

"rx-power-olt": "-21.3",

"pon-name": "PON_1_GPON",

"fiber-distance": "0.119",

"admin-status": "up",

"module-voltage": "3.3",

"name": "ONT123456789012",

"tx-bias-temperature": "28",

"presence-status": "onu-present-and-on-intended-channel-termination",

"detected-serial-number": "ALCLF81E1CAE",

"onu-id": "0"

}

],

"inventory": [

{

"oui": "-",

"hardware-revision": "10",

"supply-voltage": "3.353",

"serial-number": "SPXNCN22120MBL",

"mfg-name": "SUPERXON LTD.",

"oper-state": "enabled",

"mfg-date": "2022-03-26T00:00:00-00:00",

"admin-state": "unlocked",

"fiber-type": "single-mode",

"clei-code": "VBIUADWBAA",

"sfp-name": "PONSFP_1",

"part-number": "3FE47581BF",

"temperature": "33.3"

}

]

}

}

Screenshot for Altiplano GUI:

### Get VLAN Sub-Interface Information

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Get VLAN Sub-Interface Information | VSI Octet Count VLAN association data Bridge port vlan VLAN Association Counters | POST | {{releasename}}-altiplano-ac/rest/restconf/data/ibn:ibn/intent={{NEBS Service Name}}%23{{Customer Name}},l2-user/intent-specific-data/show-vsi-info:show-vsi-info |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{NEBS Service Name}} | The service name configured in l2-user intent. |
| {{Customer Name}} | The customer name configured in l2-user intent. |

Expected response:

{

"show-vsi-info:output": {

"vsi-info": {

"out-octets": "0",

"in-octets": "0",

"s-vlan": "1000",

"admin-status": "up",

"last-change": "2023-05-28T11:43:14+00:00",

"oper-status": "up",

"in-discards": "0",

"out-discards": "0",

"c-vlan": "100",

"qos-policy": "US_NEBS"

}

}

}

Screenshot for NC Browser GUI:

### Query VLAN Association DHCP Sessions

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Query VLAN Association DHCP Sessions | GPON VLAN Association DHCP Session dhcpSessionEntryClientIPAddress dhcpSessionEntryLeaseTime dhcpSessionEntryRemainingTime | POST | {{releasename}}-altiplano-ac/rest/restconf/data/ibn:ibn/intent={{NEBS Service Name}}%23{{Customer Name}},l2-user/intent-specific-data/anti-spoofing-action:dhcp-sessions |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{NEBS Service Name}} | The service name configured in l2-user intent. |
| {{Customer Name}} | The customer name configured in l2-user intent. |

Expected response:

{

"anti-spoofing-action:output": {

"sessions": {

"vendor-specific-tag-removed-packets-to-client-pppoe": "0",

"in-packets-from-client-pppoe": "0",

"in-packets-from-server-pppoe": "0",

"out-packets-to-client-pppoe": "0",

"out-packets-to-server-pppoe": "0",

"option-18-inserted-packets-to-server-v6": "2",

"ipv4-sessions": [

{

"prefix-length": "32",

"chaddr": "00:10:94:00:00:03",

"address-origin": "dhcp",

"expiry-time": "2023-09-19T04:23:55+00:00",

"lease-time": "3179",

"ip": "192.85.1.4"

}

],

"in-error-packets-from-server-pppoe": "0",

"in-error-packets-from-client-pppoe": "0",

"outgoing-mtu-exceeded-packets-from-client-pppoe": "0",

"out-packets-to-client-v4": "2",

"option-82-inserted-packets-to-server-v4": "2",

"option-18-removed-packets-to-client-v6": "2",

"in-bad-packets-from-client-v6": "0",

"option-37-removed-packets-to-client-v6": "0",

"vendor-specific-tag-inserted-packets-to-server-pppoe": "0",

"ipv6-sessions": [

{

"prefix-length": "128",

"chaddr": "00:10:94:00:00:05",

"address-origin": "dhcp",

"expiry-time": "",

"lease-time": "2023-10-19T03:30:16+00:00",

"ip": "2001::3"

}

],

"option-82-removed-packets-to-client-v4": "2",

"option-17-inserted-packets-to-server-v6": "0",

"option-17-removed-packets-to-client-v6": "0",

"option-82-not-inserted-to-server-v4": "0",

"option-37-inserted-packets-to-server-v6": "0",

"in-bad-packets-from-server-v6": "0",

"in-packets-from-server-v4": "2",

"in-packets-from-client-v4": "2",

"in-bad-packets-from-server-v4": "0",

"in-bad-packets-from-client-v4": "0",

"out-packets-to-server-v4": "2",

"outgoing-mtu-exceeded-packets-from-clientt-v6": "0"

}

}

}

Screenshot for NC Browser GUI:

### Query All MF OLT

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| ES DB Query | Query all MF OLT | get managed elements | POST | altiplano-elasticsearch/intents/_search/ |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Sample request body:

{

"query": {

"bool": {

"filter": [

{

"term": {

"intent-type": "device-mf"

}

}

]

}

},

"_source": [

"target",

"configuration",

"required-network-state",

"state"

]

}

Sample response body:

{

"took": 72,

"timed_out": false,

"_shards": {

"total": 95,

"successful": 95,

"skipped": 0,

"failed": 0

},

"hits": {

"total": {

"value": 12,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "intents-device-mf-v4",

"_type": "_doc",

"_id": "device-mf_$$_SWOLT0000061NK",

"_score": 0.0,

"_source": {

"required-network-state": "active",

"configuration": {

"lt-device-template": [

"default"

],

"transport-protocol": [

"ssh"

],

"ihub-device-template": [

"default"

],

"ihub-version": [

"22.9"

],

"board-service-profile": [

"Embedded-ONT",

"MDA-Profile"

],

"slot-name": [

"FAN3",

"FAN4",

"PSU2",

"LT1",

"LT2",

"ACU",

"NTIO2",

"NTIO1",

"PSU1",

"NTA",

"NTB",

"FAN1",

"FAN2"

],

"partition-access-profile": [

"PublicPAP"

],

"ip-address": [

"20.1.0.61"

],

"password": [

"$-0$uijbREnuMteC97bm5XIcwVINF0dW/hRm7L5iA/Z4Sms="

],

"device-version": [

"22.9"

],

"device-template": [

"default"

],

"push-nav-configuration-to-device": [

"true"

],

"timezone-name": [

"Australia/Sydney"

],

"planned-type": [

"LTIO-A",

"LFAN-A",

"LPWR-A",

"LANT-A",

"LWLT-C",

"LALM-A"

],

"hardware-type": [

"LS-MF-LANT-A"

],

"ip-port": [

832

],

"device-manager": [

"NAV"

],

"username": [

"admin"

]

},

"state": {

"discovered-duids": [

""

]

},

"target": {

"device-name": "SWOLT0000061NK",

"raw": "SWOLT0000061NK"

}

}

},

{

"_index": "intents-device-mf-v4",

"_type": "_doc",

"_id": "device-mf_$$_SWOLT000044",

"_score": 0.0,

"_source": {

"required-network-state": "active",

"configuration": {

"lt-device-template": [

"default"

],

"ihub-device-template": [

"qd-mf14-ihub-template"

],

"transport-protocol": [

"ssh"

],

"ihub-version": [

"22.12"

],

"board-service-profile": [

"Embedded-ONT",

"MDA-Profile"

],

"slot-name": [

"FAN3",

"FAN4",

"ACU",

"NTIO2",

"PSU2",

"NTIO1",

"LT1",

"PSU1",

"NTA",

"NTB",

"FAN1",

"FAN2"

],

"partition-access-profile": [

"PublicPAP"

],

"passive-device-version": [

"22.12"

],

"ip-address": [

"10.106.132.44"

],

"password": [

"$-0$ctiof3NBb1RY+rhbD2CgW5OtUfD9I9SPUahL5IA90uM="

],

"device-version": [

"22.12"

],

"device-template": [

"default"

],

"push-nav-configuration-to-device": [

"true"

],

"timezone-name": [

"Australia/Melbourne"

],

"planned-type": [

"LTIO-A",

"LFAN-A",

"LPWR-A",

"LWLT-C",

"LANT-A",

"LALM-A"

],

"hardware-type": [

"LS-MF-LANT-A"

],

"ip-port": [

832

],

"device-manager": [

"NAV"

],

"username": [

"admin"

]

},

"state": {

"discovered-duids": [

""

],

"host-id": null

},

"target": {

"device-name": "SWOLT000044",

"raw": "SWOLT000044"

}

}

}

]

}

}

### Query All Fiber under OLT

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| ES DB Query | Query All Fiber under OLT | Query All Fiber under OLT | GET | altiplano-elasticsearch/intents/_search/ |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Sample request body:

{

"query": {

"bool": {

"filter": [

{

"term": {

"intent-type": "fiber"

}

}

],

"must": [

{

"match": {

"configuration.device-name": "{{OLT Name}}"

}

}

]

}

},

"_source": [

"target",

"configuration",

"required-network-state",

"state"

]

}

Sample response body:

{

"took": 33,

"timed_out": false,

"_shards": {

"total": 50,

"successful": 50,

"skipped": 0,

"failed": 0

},

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.2876821,

"hits": [

{

"_index": "intents-fiber-v4",

"_type": "_doc",

"_id": "fiber_$$_SWOLT000044.L1P1",

"_score": 0.2876821,

"_source": {

"required-network-state": "active",

"configuration": {

"xpon-type": [

"mpm-gpon-xgs"

],

"device-name": [

"SWOLT000044"

],

"pon-id": [

"LT1.PON1"

],

"cage-mode": [

"auto"

],

"port-profile": [

"Default Port Profile"

],

"wavelength-profile": [

"auto"

],

"line-rate": [

"auto"

],

"model-name": [

"NOKIA MPM"

]

},

"state": null,

"target": {

"raw": "SWOLT000044.L1P1",

"fiber-name": "SWOLT000044.L1P1"

}

}

}

]

}

}

### Show Equipment Slot inventory

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent action | Query OLT slot and boards |  | POST | {{protocol}}:///{{server}}/{{base-url}}/rest/restconf/data/ibn:ibn/intent={{Device Name}},device-mf/intent-specific-data/eqpt:slot-inventory |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{Device Name}} | The Device Name is the OLT target name in device-mf intent. |

Sample response body:

{

"eqpt:output": {

"inventory": [

{

"code": "3FE75406AAAA",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Chassis",

"model": "LMFS-A",

"serial-num": "YP2248SH002"

},

{

"code": "3FE75409AAAA",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Acu",

"model": "LALM-A",

"serial-num": "YP2246SH148"

},

{

"code": "3FE76353AABA",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-LT1",

"model": "LWLT-C",

"serial-num": "YP2241F5515"

},

{

"code": "3FE74868AAAB",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Nta",

"model": "LANT-A",

"serial-num": "YP2239F44AA"

},

{

"code": "3FE74868AAAB",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Ntb",

"model": "LANT-A",

"serial-num": "YP2239F44BE"

},

{

"code": "3FE75407ABAA",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Fan1",

"model": "LFAN-A",

"serial-num": "YP2246SH032"

},

{

"code": "3FE75407ABAA",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Fan2",

"model": "LFAN-A",

"serial-num": "YP2246SH04C"

},

{

"code": "3FE75407ABAA",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Fan3",

"model": "LFAN-A",

"serial-num": "YP2246SH051"

},

{

"code": "3FE75407ABAA",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Fan4",

"model": "LFAN-A",

"serial-num": "YP2246SH04D"

},

{

"code": "3FE75408BAAA",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Psu1",

"model": "LPWR-A",

"serial-num": "YP2246SH0D3"

},

{

"code": "3FE75408BAAA",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Psu2",

"model": "LPWR-A",

"serial-num": "YP2246SH0D4"

},

{

"code": "3FE77199AAAA",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Ntio1",

"model": "LTIO-A",

"serial-num": "YP2243F44CE"

},

{

"code": "3FE77199AAAA",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Ntio2",

"model": "LTIO-A",

"serial-num": "YP2243F44D6"

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Acu",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-1",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-2",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-3",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-4",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-5",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-6",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-7",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-8",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-9",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Nt-A",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Nt-B",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Fan-1",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Fan-2",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Fan-3",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Fan-4",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-10",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-11",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-12",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-13",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Lt-14",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Psu-1",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Psu-2",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Ntio-1",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Slot-Ntio-2",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Nta.CPU1",

"model": "",

"serial-num": ""

},

{

"code": "",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Nta.NVM1",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Acu.TEMP1",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Nta.TEMP1",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Nta.TEMP2",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Nta.TEMP3",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Nta.TEMP4",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Nta.TEMP5",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Nta.TEMP6",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Nta.TEMP7",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Nta.TEMP8",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Ntb.TEMP1",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Ntb.TEMP2",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Ntb.TEMP3",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Ntb.TEMP4",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Ntb.TEMP5",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Ntb.TEMP6",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Ntb.TEMP7",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Ntb.TEMP8",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Ntio1.TEMP1",

"model": "",

"serial-num": ""

},

{

"code": "temperature sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Ntio2.TEMP1",

"model": "",

"serial-num": ""

},

{

"code": "Fan Speed sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Fan1.FANSPEED1",

"model": "",

"serial-num": ""

},

{

"code": "Fan Speed sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Fan1.FANSPEED2",

"model": "",

"serial-num": ""

},

{

"code": "Fan Speed sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Fan2.FANSPEED1",

"model": "",

"serial-num": ""

},

{

"code": "Fan Speed sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Fan2.FANSPEED2",

"model": "",

"serial-num": ""

},

{

"code": "Fan Speed sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Fan3.FANSPEED1",

"model": "",

"serial-num": ""

},

{

"code": "Fan Speed sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Fan3.FANSPEED2",

"model": "",

"serial-num": ""

},

{

"code": "Fan Speed sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Fan4.FANSPEED1",

"model": "",

"serial-num": ""

},

{

"code": "Fan Speed sensor",

"admin-status": "unlocked",

"oper-status": "enabled",

"name": "Board-Fan4.FANSPEED2",

"model": "",

"serial-num": ""

}

]

}

}

AAC gui snapshot:

### Enable/Disable PON Utilization Monitoring

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Enable PON Utilization Monitoring | Fiber Optics measurement | POST | {{protocol}}:///{{server}}/{{base-url}}/rest/restconf/data/ibn:ibn/intent={{Fiber Name}},fiber/intent-specific-data/enable-disable-speed-monitoring:enable-speed-monitoring |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{Fiber Name}} | The Fiber name configured in fiber intent. SWOLTxxxxxx.Lyy.Pzz​ e.g.​ SWOLT0000333.L13.P05 |

### Show FDB Mac Address

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Show FDB Mac |  | POST | {{protocol}}:///{{server}}/{{base-url}}/rest/restconf/data/ibn:ibn/intent={{NEBS Service Name}}%23{{Customer Name}},l2-user/intent-specific-data/show-fdb-mac:show-fdb-mac |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{NEBS Service Name}} | The service name configured in l2-user intent. |
| {{Customer Name}} | The customer name configured in l2-user intent. |

Sample response body:

{

"show-fdb-mac:output": {

"fdb-table": [

{

"mac": "00:00:01:00:00:09"

},

{

"mac": "00:00:01:00:00:0e"

},

{

"mac": "00:00:01:00:00:15"

},

{

"mac": "00:00:01:00:00:1b"

},

{

"mac": "00:00:01:00:00:1c"

},

{

"mac": "00:00:01:00:00:2c"

},

{

"mac": "00:00:01:00:00:36"

},

{

"mac": "00:00:01:00:00:3b"

}

]

}

}

Screenshot for Altiplano GUI:

### Show Uplink Port SFP Diagnostics and Inventory

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Show SFP Diagnostics and Inventory |  | POST | {{protocol}}:///{{server}}/{{base-url}}/rest/restconf/data/ibn:ibn/intent={{Uplink Connection Name}},uplink-connection/intent-specific-data/sfp-status:show-sfp-diagnostics-and-inventory |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{ Uplink Connection Name }} | The Fiber name configured in uplink-connection intent. |
| {{ Uplink Port Name}} | The pattern of {{ Uplink Port Name }} is like “nt-a:xfp:1” or “ltio-a-1:qsfp:1-1” or “ntio-1:qsfp:1-1” |

Sample request body:

{

"sfp-status:input": {

"entity": "{{ Uplink Port Name}}"

}

}

Sample response body:

{

"sfp-status:output": {

"sfp-status": [

{

"tx-power": "0.29",

"loss-of-signal": "",

"supply-voltage": "3.25",

"rx-power": "-1.09",

"sfp-temp": "28.6",

"bias-current": "32.7",

"name": "ntio-1:qsfp:1-1",

"tx-failed": "",

"availability": ""

}

],

"tca-status": [

{

"tx-power": "Normal",

"rx-power": "Normal",

"bias-current": "Normal",

"name": "ntio-1:qsfp:1-1",

"temperature": "Normal",

"voltage": "Normal"

}

],

"inventory": [

{

"manufacture-date": "2022-09-27T00:00:00.0Z",

"serial-number": "HGGNCN22352536  ",

"additional-info": "MTRQ-4PA00      ",

"oui-code": "da:28:ec",

"oper-state": "up",

"manufacture-name": "",

"admin-state": "enable",

"fiber-type": "",

"clei-code": "",

"wave-length": "",

"part-number": "3FE75086AA  ALCATEL       ",

"name": "ntio-1:qsfp:1-1",

"basic-type": "",

"inv-availability": ""

}

]

}

}

Screenshot for Altiplano GUI:

### Restart

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Restart |  | POST | {{releasename}}-altiplano-ac/ rest/restconf/data/ibn:ibn/intent={{Device Name}},device-mf/intent-specific-data/restart:restart |

Headers:

| Key | Value |
| --- | --- |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{ Device Name}} | The Device MF target name, eg SWOLT000044 |
| {{Restart Mode}} | Restart Mode: with-self-test without-self-test hot-restart to-default-configuration |
| {{Slot Name}} | The pattern of {{ Slot Name }} is like: Board-LT1, Board-Nta, Board-Ntb, Chassis |

Sample request body:

{

"restart:input": {

"entity": "{{Slot Name}}",

"restart": "{{Restart Mode}}"

}

}

Expected respond XML result:

Respond 204 No Content

### Show Last Restart Time of Slot

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Show Last Restart Time |  | POST | {{protocol}}:///{{server}}/{{base-url}}/rest/restconf/data/ibn:ibn/intent={{Device Name}},device-mf/intent-specific-data/eqpt:show-last-restart-time |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{ Device Name}} | The device name configured in device-mf intent. |
| {{ Card Name}} | LT “Slot Name” configured in corresponding device-mf intent,like “LT1”,”LT2”and so on |

Sample request body:

{

"eqpt:input": {

"card-name": {{Card Name}}

}

}

Sample response body:

{

"eqpt:output": {

"inventory": [

{

"name": "SWOLT000044.LT1",

"last-restart-time": "2023-10-20T01:07:41+00:00"

}

]

}

}

Screenshot for Altiplano GUI:

### Show  NT Protection State

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Show NT Protection State |  | POST | {{protocol}}:///{{server}}/{{base-url}}/rest/restconf/data/ibn:ibn/intent={{Device Name}},device-mf/intent-specific-data/eqpt:nt-protection-state |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{ Device Name}} | The device name configured in device-mf intent. |

Sample request body:

Null

Sample response body:

{

"eqpt:output": {

"inventory": [

{

"troubleshooting-info": "All NEs available",

"standby-state": "HOT-STANDBY",

"name": "Board-Nta",

"standby-state-change-reason": "NONE",

"group": "NT-Protection",

"standby-state-change-time": "2023-10-26T12:55:24+11:00"

},

{

"troubleshooting-info": "Active nt",

"standby-state": "PROVIDING-SERVICE",

"name": "Board-Ntb",

"standby-state-change-reason": "NONE",

"group": "NT-Protection",

"standby-state-change-time": "2023-10-20T12:56:25+11:00"

}

]

}

}

Screenshot for Altiplano GUI:

### Show Forwording Database MAC

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Show Forwording Database MAC |  | POST | {{protocol}}:///{{server}}/{{base-url}}/rest/restconf/data/ibn:ibn/intent={{Device Name}},device-mf/intent-specific-data/eqpt:show-forwarding-databases-mac |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{ Device Name}} | The device name configured in device-mf intent. |
| {{ Card Name}} | LT “Slot Name” configured in corresponding device-mf intent,like “LT1”,”LT2”and so on |

Sample request body:

{

"eqpt:input": {

"card-name": {{Card Name}}

}

}

Sample response body:

{

"eqpt:output": {

"inventory": [

{

"forwarder-port": "USR_PORT_ont1_LAN1_NEBS",

"forwarder": "FWD_NEBS_ont1",

"sub-interface": "-",

"mac": "00:00:01:04:00:01"

},

{

"forwarder-port": "USR_PORT_ont1_LAN1_NEBS",

"forwarder": "FWD_NEBS_ont1",

"sub-interface": "-",

"mac": "00:00:01:05:00:01"

},

{

"forwarder-port": "USR_PORT_ont1_LAN1_NEBS",

"forwarder": "FWD_NEBS_ont1",

"sub-interface": "-",

"mac": "00:02:01:00:00:02"

}

]

}

}

Screenshot for Altiplano GUI:

### Show  CFM MAC Per Board

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Show CFM MAC Per Board |  | POST | {{protocol}}:///{{server}}/{{base-url}}/rest/restconf/data/ibn:ibn/intent={{Device Name}},device-mf/intent-specific-data/eqpt:show-cfm-mac-per-board |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{ Device Name}} | The device name configured in device-mf intent. |
| {{ Card Name}} | LT “Slot Name” configured in corresponding device-mf intent,like “LT1”,”LT2”and so on |

Sample request body:

{

"eqpt:input": {

"card-name": {{Card Name}}

}

}

Sample response body:

{

"eqpt:output": {

"inventory": [

{

"service-id": "20",

"port": "ENET_LAN1",

"onu-name": "ont1",

"mac": "70:d9:31:6c:96:bc",

"md-id": "-"

},

{

"service-id": "30",

"port": "ENET_LAN1",

"onu-name": "ont1",

"mac": "70:d9:31:6c:96:bc",

"md-id": "-"

},

{

"service-id": "40",

"port": "ENET_LAN1",

"onu-name": "ont1",

"mac": "70:d9:31:6c:96:bc",

"md-id": "-"

}

]

}

}

Screenshot for Altiplano GUI:

### Show Optics Inventory Information

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Show Optics Inventory Information |  | POST | {{protocol}}:///{{server}}/{{base-url}}/rest/restconf/data/ibn:ibn/intent={{Device Name}},device-mf/intent-specific-data/eqpt-optics-action:optics-inventory |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{ Device Name}} | The device name configured in device-mf intent. |
| {{ Card Name}} | "NTIO" or specified LT name ("NTIO" to retrieve uplink port info. LT name e.g "LT1" to retrieve PON port info of the specified LT) |

Sample request body:

{

"eqpt-optics-action:input": {

"card-name": {{Card Name}}

}

}

Sample response body:

{

"eqpt-optics-action:output": {

"inventory": [

{

"rxpower": "-",

"txpower": "6.1",

"name": "PON_1_XGS",

"serial-num": "SPXNCN22120MBL"

},

{

"rxpower": "-",

"txpower": "5.2",

"name": "PON_1_GPON",

"serial-num": "SPXNCN22120MBL"

},

{

"rxpower": "-",

"txpower": "-",

"name": "PONSFP_1",

"serial-num": "SPXNCN22120MBL"

}

]

}

}

Screenshot for Altiplano GUI:

### Show LAG Statistics

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Show LAG Statistics |  | POST | {{protocol}}:///{{server}}/{{base-url}}/rest/restconf/data/ibn:ibn/intent={{Uplink Connection Name }},uplink-connection/intent-specific-data/uplink-lag-action:lag-statistics |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{ Uplink Connection Name }} | The name configured in uplink-connection intent. |

Sample request body:

null

Sample response body:

{

"uplink-lag-action:output": {

"inventory": [

{

"total-in-errors": "0",

"lacp-rx-illegal": "0",

"lacp-rx-errors": "0",

"total-in-octets": "290922085128",

"total-out-octets": "169380824096",

"lag-id": "1",

"total-out-packets": "112654949",

"port-id": "1/6/c1/1",

"total-out-errors": "0",

"total-in-packets": "1163641151",

"lacp-rx-packets": "10242",

"lacp-tx-packets": "37952"

},

{

"total-in-errors": "0",

"lacp-rx-illegal": "0",

"lacp-rx-errors": "0",

"total-in-octets": "0",

"total-out-octets": "0",

"lag-id": "1",

"total-out-packets": "0",

"port-id": "1/7/c1/1",

"total-out-errors": "0",

"total-in-packets": "0",

"lacp-rx-packets": "0",

"lacp-tx-packets": "0"

}

]

}

}

Screenshot for Altiplano GUI:

## RC-Proxy query

Most of the Test & Diagnosis CMD are Intent Actions, or Intent query. This is aligning to the concept of Intent Based Networking.

Eventually, Intent is an abstract layer. We can expose most of the regular used YANG to Intent. “regular” means such YANG is frequently modified, queried from OSS provisioning or T&D system.

But, Intent can not cover all the device YANG details. Otherwise, it is not an abstract layer anyway. In case operator want to access some “non-regular” YANG, Altiplano provides the application, “NC Browser”, to view/config these YANGs.

In case the request is coming from north bound, instead of Altiplano GUI, Restful CMD is to be used. Since there maybe many AAV/NV instances in production, it will be difficult for OSS to allocate which AAV/NV of one particular device. RC-Proxy is the micro-service to solve the device to AAV/NV mapping. The OSS will only query RC-Proxy, no matter which device. It is up to the RC-Proxy to dispatch the query to the right AAV/NV. This kind of query, we call “RC-Proxy” query.

### NC Browser

#### Select the right device

- Select the right AAV/NV instance from “control/virtulizer”

- Select the “device-manager”

- In the “anv-device-holders:device” drop down window, select the right device

#### Select specific YANG

- Select the “device-specific-data”

- Select the right YANG to view/configure

### RC-Proxy query Url

Common Url:

{{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/{{YANG-path}}

Sample 1: query ONT interface

{{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/ietf-interfaces:interfaces-state/interface={{ONT Name}}

Sample 2: query ONT Software Version

{{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/bbf-fiber-onu-emulated-mount:onus/onu={{ONT Name}}/root/ietf-hardware-mounted:hardware-state/component=CHASSIS/bbf-software-image-management-one-dot-one-mounted:software/software=application_software

### ONT Admin status / Operational status / Last change date / Ranged / SN / Distance

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| RC device proxy | Admin status / Operational status / Last change date / Ranged / SN / Distance | Get NTD Status | GET | {{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/ietf-interfaces:interfaces-state/interface={{ONT Name}} |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Expected respond XML result:

{

"ietf-interfaces:interface": {

"lower-layer-if": [

"SWOLT000044.L1P1_CPART_GPON"

],

"admin-status": "up",

"last-change": "2023-03-21T09:30:25+00:00",

"bbf-xponvani:v-ani": {

"onu-present-on-this-olt": {

"onu-fiber-distance": 115,

"detected-registration-id": "44454641554c54",

"detected-serial-number": "ALCLF81E1CAE"

},

"management-tcont-alloc-id": 1,

"onu-presence-state": "bbf-xpon-onu-types:onu-present-and-on-intended-channel-termination",

"nokia-sdan-if-xponvani-aug:onu-last-down-reason": "bbf-xpon-onu-types:onu-not-present",

"management-gemport-id": 1,

"onu-id": 1

},

"oper-status": "up",

"name": "ONT123456789012",

"higher-layer-if": [

"VENET_ONT123456789012_LAN1"

],

"type": "bbf-xpon-if-type:v-ani",

"if-index": 520,

"statistics": {

"out-octets": "0",

"in-octets": "0",

"bbf-interfaces-statistics-management:in-pkts": "0",

"nokia-sdan-additional-statistics:out-dropped-bytes": "0",

"bbf-interfaces-statistics-management:out-pkts": "0",

"nokia-sdan-additional-statistics:in-dropped-bytes": "0",

"discontinuity-time": "2023-03-21T09:30:22+00:00",

"in-discards": 0,

"out-discards": 0

}

}

}

### ONT Software Version

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| RC device proxy | ONT Software Version | Get NTD Status | GET | {{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/bbf-fiber-onu-emulated-mount:onus/onu={{ONT Name}}/root/ietf-hardware-mounted:hardware-state/component=CHASSIS/bbf-software-image-management-one-dot-one-mounted:software/software=application_software |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Expected respond XML result:

{

"bbf-software-image-management-one-dot-one-mounted:software": {

"name": "application_software",

"revisions": {

"revision": [

{

"is-valid": true,

"name": "3FE56161AOCK35",

"is-committed": false,

"is-active": false,

"version": "3FE56161AOCK35"

},

{

"is-valid": true,

"name": "3FE56161AOCK37",

"is-committed": true,

"is-active": true,

"version": "3FE56161AOCK37"

}

]

}

}

}

### ONT Optical Power

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| RC device proxy | ONT Optical Power | Get NTD Status | GET | {{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/bbf-fiber-onu-emulated-mount:onus/onu={{ONT Name}}/root/ietf-hardware-mounted:hardware-state/component=ANIPORT |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Expected respond XML result:

{

"ietf-hardware-mounted:component": {

"bbf-hardware-transceivers-mounted:transceiver-link": {

"diagnostics": {

"tx-power": 19,

"nokia-hardware-transceivers-dbm-mounted:tx-power-dbm": 30,

"tx-bias": 7127,

"rx-power": 0,

"nokia-hardware-transceivers-dbm-mounted:rx-power-dbm": -172,

"laser-temperature": 8192

}

},

"name": "ANIPORT",

"class": "bbf-hardware-types:transceiver-link"

}

}

### ONT Eth Card / Port State

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| RC device proxy | ONT Optical Power | Get UNI-D Status | GET | {{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/bbf-fiber-onu-emulated-mount:onus/onu={{ONT Name}}/root/ietf-interfaces-mounted:interfaces-state/interface=ENET_{{LAN Name}} |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{LAN Name}} | The pattern of {{LAN Name}} is like “LAN1” or “10GE” |

Expected respond XML result:

{

"ietf-interfaces-mounted:interface": {

"admin-status": "up",

"oper-status": "up",

"name": "ENET_LAN1",

"phys-address": "00:00:00:00:00:00",

"ethernet-mounted:Ethernet": {

"nokia-ethernet-interface-fec-mounted:fec": "per-802.3",

"duplex": "full",

"speed": "1000"

},

"type": "iana-if-type-mounted:ethernetCsmacd",

"speed": "1000000000"

}

}

### ONT Eth Auto Detection Configuration

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| RC device proxy | ONT Eth Auto Detection Configuration | bponLanEtherPortAutoDetectConfig | GET | {{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/bbf-fiber-onu-emulated-mount:onus/onu={{ONT Template Name}}/root/ietf-interfaces-mounted:interfaces-state/interface=ENET_{{LAN Name}} |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{LAN Name}} | The pattern of {{LAN Name}} is like “LAN1” or “10GE” |

Expected respond XML result:

TBD – check <ethernet> element field

### ONT Eth Port Actual Speed

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| RC device proxy | ONT Eth Port Actual Speed | bponLanEtherPortOpstatConfigIndicator | GET | {{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/bbf-fiber-onu-emulated-mount:onus/onu={{ONT Name}}/root/ietf-interfaces-mounted:interfaces-state/interface=ENET_{{LAN Name}} |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{LAN Name}} | The pattern of {{LAN Name}} is like “LAN1” or “10GE” |

Expected respond XML result:

TBD – check <ethernet> element field

### VLAN Association Data

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| RC device proxy | VLAN association data | Get UNI-D Status | GET | {{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/bbf-fiber-onu-emulated-mount:onus/onu={{ONT Name}}/nokia-onus-from-template:template-parameters/interfaces/interface=VSI_{{LAN Name}}_{{Service Name}} |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{LAN Name}} | The pattern of {{LAN Name}} is like “LAN1” or “10GE” |
| {{Service Name}} | This is the service name configured in related onu-template. Should be NEBS here. |

Expected respond XML result:

{

"nokia-onus-from-template:interface": {

"template-ref": "VSI_LAN1_NEBS",

"ipv4": {

"origin": "static"

},

"pm-enable": false,

"vsi": {

"ingress-rule": [

{

"name": "single-tagged",

"match-criteria-tag-0-vlan-id": "100",

"ingress-rewrite-tag-0-vlan-id": 100

}

],

"ingress-qos-policy-profile": "US_NEBS"

}

}

}

### Bridge Port Vlan

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| RC device proxy | Bridge Port Vlan | Get UNI-D Status | GET | {{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/ietf-interfaces:interfaces/interface=VSI_{{ONT Name}}_{{LAN Name}}_{{Service Name}}/bbf-frame-processing-profile:tag-0 |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{LAN Name}} | The pattern of {{LAN Name}} is like “LAN1” or “10GE” |
| {{Service Name}} | This is the service name configured in related onu-template. Should be NEBS here. |

Expected respond XML result:

{

"bbf-frame-processing-profile:tag-0": {

"vlan-id": 100

}

}

### NTD UNI-D Counters

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| RC device proxy | NTD UNI-D Counters | Octet Count | GET | {{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/ietf-interfaces:interfaces-state/interface=VENET_{{ONT Name}} _{{UNI Name}} /statistics |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{UNI Name}} | The pattern of {{UNI Name}} is like “LAN1” or “10GE” |

Expected respond XML result:

{

"ietf-interfaces:statistics": {

"out-octets": "0",

"in-octets": "0",

"discontinuity-time": "2023-03-21T09:30:22+00:00",

"in-discards": 0,

"out-discards": 0

}

}

### VLAN Association Counters

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| RC device proxy | VLAN Association Counters | Octet Count | GET | {{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/ietf-interfaces:interfaces-state/interface= VSI_{{ONT Name}}_{{LAN Name}}_{{Service Name}}/statistics |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{LAN Name}} | The pattern of {{LAN Name}} is like “LAN1” or “10GE” |
| {{Service Name}} | This is the service name configured in related onu-template. Should be NEBS here. |

Expected respond XML result:

{

"ietf-interfaces:statistics": {

"nokia-interfaces-pktcnt:out-pkts": "0",

"out-octets": "0",

"in-octets": "0",

"bbf-l2-dhcpv4-relay:dhcpv4": {

"in-packets-from-client": 0,

"out-packets-to-client": 0,

"option-82-inserted-packets-to-server": 0,

"option-82-removed-packets-to-client": 0,

"in-bad-packets-from-server": 0,

"in-bad-packets-from-client": 0,

"in-packets-from-server": 0,

"out-packets-to-server": 0,

"option-82-not-inserted-to-server": 0

},

"discontinuity-time": "2023-03-21T09:52:53+00:00",

"bbf-icmpv6:icmpv6-stats": {

"upstream": {

"neighbour-solicitation": {

"discard-packets": 0,

"forward-packets": 0

},

"router-advertisement": {

"discard-packets": 0,

"forward-packets": 0

},

"multicast-listener-query": {

"discard-packets": 0,

"forward-packets": 0

},

"router-redirect": {

"discard-packets": 0,

"forward-packets": 0

},

"error-message": {

"discard-packets": 0,

"forward-packets": 0

},

"invalid-message": {

"discard-packets": 0,

"forward-packets": 0

},

"neighbour-advertisement": {

"discard-packets": 0,

"forward-packets": 0

},

"informational-message": {

"discard-packets": 0,

"forward-packets": 0

},

"router-solicitation": {

"discard-packets": 0,

"forward-packets": 0

}

},

"downstream": {

"neighbour-solicitation": {

"discard-packets": 0,

"forward-packets": 0

},

"router-advertisement": {

"discard-packets": 0,

"forward-packets": 0

},

"multicast-listener-query": {

"discard-packets": 0,

"forward-packets": 0

},

"router-redirect": {

"discard-packets": 0,

"forward-packets": 0

},

"error-message": {

"discard-packets": 0,

"forward-packets": 0

},

"invalid-message": {

"discard-packets": 0,

"forward-packets": 0

},

"neighbour-advertisement": {

"discard-packets": 0,

"forward-packets": 0

},

"informational-message": {

"discard-packets": 0,

"forward-packets": 0

},

"router-solicitation": {

"discard-packets": 0,

"forward-packets": 0

}

}

},

"in-discards": 0,

"out-discards": 0,

"nokia-interfaces-pktcnt:in-pkts": "0",

"bbf-ldra:dhcpv6-ldra": {

"outgoing-mtu-exceeded-packets-from-client": 0,

"option-18-inserted-packets-to-server": 0,

"option-17-removed-packets-to-client": 0,

"option-37-removed-packets-to-client": 0,

"option-37-inserted-packets-to-server": 0,

"option-17-inserted-packets-to-server": 0,

"option-18-removed-packets-to-client": 0,

"in-bad-packets-from-server": 0,

"in-bad-packets-from-client": 0

},

"bbf-pppoe-intermediate-agent:pppoe": {

"outgoing-mtu-exceeded-packets-from-client": 0,

"vendor-specific-tag-removed-packets-to-client": 0,

"in-packets-from-client": 0,

"in-error-packets-from-server": 0,

"out-packets-to-client": 0,

"in-packets-from-server": 0,

"in-error-packets-from-client": 0,

"out-packets-to-server": 0,

"vendor-specific-tag-inserted-packets-to-server": 0

}

}

}

### PON Admin / Operational status

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| RC device proxy | PON Admin / Operational status | PON Status | GET | {{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/ietf-interfaces:interfaces-state/interface={{Fiber Name}} |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{Fiber Name}} | The fiber name configured in fiber intent. E.g.: SWOLT000044.L1P1 |

Expected respond XML result:

{

"ietf-interfaces:interface": {

"admin-status": "up",

"last-change": "2023-03-21T05:17:04+00:00",

"oper-status": "up",

"bbf-xpon:channel-group": {

"allocated-upstream-channel-ids": {

"channel-id": [

"xgs"

]

},

"allocated-downstream-channel-ids": {

"downstream-channel-id": [

"xgs"

]

},

"allocated-downstream-wavelengths": {

"wavelength": [

"xgs"

]

}

},

"name": "SWOLT000044.L1P1",

"higher-layer-if": [

"SWOLT000044.L1P1_CPART_GPON",

"SWOLT000044.L1P1_CPART_XGS"

],

"type": "bbf-xpon-if-type:channel-group",

"if-index": 522

}

}

### PON optical data

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| RC device proxy | PON optical data | PON optical data | GET | {{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/ietf-hardware:hardware-state/component={{PON ID}}_[ GPON ¦ XGS ]/transceiver-link |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{PON ID}}_[ GPON ¦ XGS ] | The pattern of {{PON ID}} is like “PON_1”. The [ GPON ¦ XGS ] is “GPON” or “XGS” Full string example: PON_1_GPON |

Expected respond XML result:

{

"transceiver-link": {

"wavelength": "1490",

"diagnostics": {

"tx-bias": 6169,

"tx-power-dbm": 52,

"rx-power-dbm": -400,

"rssi-onu": {

"detected-serial-number": "ALCLF81E1CAE",

"rssi": "-209",

"v-ani-ref": "ONT123456789012"

}

}

}

}

### Query VLAN Association DHCP Sessions

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| RC device proxy | Query VLAN Association DHCP Sessions | GPON VLAN Association DHCP Session dhcpSessionEntryClientIPAddress dhcpSessionEntryLeaseTime dhcpSessionEntryRemainingTime | GET | {{releasename}}-altiplano-rcdeviceproxy/rest/restconf/data/anv:device-manager/anv-device-holders:device={{LT Device Name}}/device-specific-data/ietf-interfaces:interfaces-state/interface= VSI_{{ONT Name}}_{{LAN Name}}_{{Service Name}}_{{AVC ID}}/ipv4-security |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{LAN Name}} | The pattern of {{LAN Name}} following example of “LAN1”. |
| {{Service Name}} | The service name in onu-template, should be NEBS here |
| {{AVC ID}} | AVC ID. |

Expected respond XML result:

{

"ipv4-security": {

"address": {

"ip": "192.85.1.4",

"prefix-length": 32,

"ip-address-origin": "dhcp",

"lease-time": 3600,

"ip-address-expiry-time": "2023-04-06T08:07:20+00:00",

"chaddr": "00:10:94:00:00:02"

}

}

}

## PM interface

### PM counter collection mode

For the performance counter, there are several collection models. Live and Bulk are widely used on daily operation and maintenance, ss shown in Altiplano GUI below:

- Live collection.

  - It is for trouble-shooting purpose. When OSS send a PM counter ad-hoc query, the Altiplano will translate to an NC/Y Get RPC to Device. Another approach to get live collection on specific is from altiplano Network view. Please refer to below samples.

  - As we can see, live collection requires to specific the device (interface, port), and interested PM counter YANG to be monitored. It is not suitable for a network wide periodic collection.

  - For the benefit of OSS (SMP) TnD easy integration, we developed/customized intent actions, which simplify the TnD ad-hoc query. Intent action groups multiple YANG elements under the same intent logical device, such as fiber, ONT, L2-user. Single query can get multiple YANG status, including admin-status, oper-status and some PM counters. Please refer to chapter 4.4 for the details.

- Bulk collection.

  - It is used for regular (eg. every 15 Mins) collection of the whole network wide

  - Ipfix is the module sitting inside the OLT device to push the counters to Altiplano DB. (Not like Live Collection, which is NC/Y get from Altiplano to OLT.)

  - Altiplano Open-TSDB is used to store the counters from the OLTs.

Note: We will focus on bulk collection setup and configuration in later sections.

### Which counters to be collected

In today’s ISAM network, below PM counters of access network points are collected

- Uplink point:

  - 15 mins Uplink out Octet

  - 15 mins Uplink out pkts

  - 15mins uplink out drop pkts

- PON point:

  - 5 mins TX total utilization

  - 5 mins RX total utilization

  - 5 mins TX total bytes

  - 5 mins RX total bytes

  - 5 mins TX total drop bytes

  - 5 mins RX total drop bytes

- VSI (VLAN sub-interface)

  - 15 mins Learned MAC on Forwarder

  - 15 mins VLAN Port association (AVC_ID)

- ONT

  - 24 hours ONT optics TX/RX power(dBM)

Note: ONT optics TX/RX power(dBM) query is a slow process, because such information is via OMCI channel. Suppose we have 2000+ ONT under one OLT, 15mins may not be good enough to get all the ONT data.

Here we setup 24 hours interval, to make sure we can have one day snapshot for inventory purposes.

For the debug and troubleshooting purposes, IBN action (Chapter 4.4.3 and 4.4.9) provides the live-collection method.

### IPFIX related profile

To enable above counters in Lightspan OLT, we need to customize the IPFIX related profiles, and push to the OLTs via device-config-mf intent.

Two IPFIX related profiles:

- Ipfix profile.

- Ipfix cache profile

#### IPFIX profile

It defines the exporting interval variables, such as “CONFIGURABLE”, “15-MINS”. These variables can be used in ipfix cache profile.

In below ipfix cache profile, we did not use the variable defined in IPFIX profile. Instead, we use direct number (in seconds)

#### IPFIX Cache profile

There are multiple IPFIX cache profiles, linking to different Device elements.

For the TRUE use cases, we should enable below counters

Ls-MF:

- xpon-interfaces-state

- xpon-interfaces-state-statistics-channel-termination

- xpon-forwarding-database

- ont-transceiver link

- xpon-interfaces-state-statistics-capacity

Ls-IHUB

- Port-pm

##### PON port octets

Select ipfix cache profile/LS-MF/xpon-interfaces-state-statictics-channel-termination

Define the scope: interface Type = channel-termination

Export internal: 300 second

State: enabled

Cache Layout Type:

- If: name

- If: in-octets

- If: out-octets

- If: in-discards

- If: out-discards

- Nokia-add-stats: in-dropped-bytes

- Nokia-add-stats: out-dropped-bytes

- bbf-if-ctrs:in-pkts

- bbf-if-ctrs:out-pkts

##### PON port admin/oper status

Select ipfix cache profile/LS-MF/xpon-interfaces-state

Define the scope: interface Type = channel-termination

Export internal: 300 second

State: enabled

Cache Layout Type:

- If: name

- If: admin-status

- If: oper-status

##### PON utilization

Support PON Utilization on network view.

Noted: To get PON Utilization, Enable PON Utilization Monitoring is required.

###### Enable/Disable PON Utilization Monitoring

| Catalog | Function | SMP CMD | METHOD | endpoint |
| --- | --- | --- | --- | --- |
| Intent Action | Enable PON Utilization Monitoring | Fiber Optics measurement | POST | {{protocol}}:///{{server}}/{{base-url}}/rest/restconf/data/ibn:ibn/intent={{Fiber Name}},fiber/intent-specific-data/enable-disable-speed-monitoring:enable-speed-monitoring |

Headers:

| Key | Value |
| --- | --- |
| Accept | application/yang-data+json |
| Authorization | JWT token |

Main Parameter:

| Variable & Parameter | Value |
| --- | --- |
| {{Fiber Name}} | The Fiber name configured in fiber intent. SWOLTxxxxxx.Lyy.Pzz​ e.g.​ SWOLT0000333.L13.P05 |

###### IPFIX configure for PON utilization

PON Utilization

Sample output(Kafka)

kubectl exec -it  altiplano-kafka-0 bash -c ckaf-kafka-broker

cp /etc/kafka/kafka.properties /etc/kafka/kafka-test.properties

java -cp /etc/kafka/shared/encrypter.jar com.nokia.fileencrypter.Decrypter /etc/kafka/shared/.secretKey /etc/kafka/kafka-test.properties

./bin/kafka-console-consumer --bootstrap-server localhost:9093 --consumer.config /etc/kafka/kafka-test.properties --topic KPI-XPON --from-beginning

##### ONT optics power

Select ipfix cache profile/LS-MF/ont-tranceiver-link

Export internal: 43200 second

State: enabled

Note: export internal to 43200 second (12 hours). It is to make sure inventory DB can get ONT optics power updates evert 24 hours.

##### Uplink port

Select ipfix cache profile/LS-IHUB/Port-pm

Export internal: 900 second

State: enabled

Cache Layout Type: port-pm

##### BER Counters

The BIP related counters are for Gpon ONTs Similar to using ISAM. The FEC counters will be for the SFP ONT G-010S-Q and not for legacy NTD.

Select ipfix cache profile/LS-MF/eout-fec-bip-pm

Select ipfix cache profile/LS-MF/ xpon-fec-bip-pm-ont

Select ipfix cache profile/LS-MF/ xpon-fec-bip-pm-pon

Sample output(Kafka)

kubectl exec -it  altiplano-kafka-0 bash -c ckaf-kafka-broker

cp /etc/kafka/kafka.properties /etc/kafka/kafka-test.properties

java -cp /etc/kafka/shared/encrypter.jar com.nokia.fileencrypter.Decrypter /etc/kafka/shared/.secretKey /etc/kafka/kafka-test.properties

./bin/kafka-console-consumer --bootstrap-server localhost:9093 --consumer.config /etc/kafka/kafka-test.properties --topic IPFIX-XPON --from-beginning

channel termination( PON  US):

{

"anv:device-manager": {

"anv-device-holders:device": [

{

"device-id": "SWOLT000044.LT1",

"timestamp": "2023-09-10T23:15:00+0000",

"device-specific-data": {

"ietf-interfaces:interfaces-state": {

"interface": [

{

"name": "CT_SWOLT000044.L1P1_1_GPON",

"bbf-interfaces-performance-management:performance": {

"intervals-15min": {

"history": [

{

"interval-number": "1",

"bbf-xpon-performance-management:xpon": {

"phy": {

"in-bip-protected-words": "0",

"in-bip-errors": "0"

}

}

}

]

}

}

}

]

}

}

}

]

}

}

ONT Uptream:

{

"anv:device-manager": {

"anv-device-holders:device": [

{

"device-id": "SWOLT000044.LT1",

"timestamp": "2023-09-10T23:15:00+0000",

"device-specific-data": {

"ietf-interfaces:interfaces-state": {

"interface": [

{

"name": "ONT123456789012",

"bbf-interfaces-performance-management:performance": {

"intervals-15min": {

"history": [

{

"interval-number": "1",

"bbf-xpon-performance-management:xpon": {

"phy": {

"in-bip-protected-words": "0",

"in-bip-errors": "0"

}

}

}

]

}

}

}

]

}

}

}

]

}

}

ONT Downstream:   the behavior depends on ONT type and version

{

"anv:device-manager": {

"anv-device-holders:device": [

{

"device-id": "Demo1",

"timestamp": "2023-10-30T12:01:00+0000",

"device-specific-data": {

"bbf-fiber-onu-emulated-mount:onus": {

"onu": [

{

"name": "Demo1_PON1_demo22",

"root": {

"ietf-interfaces-mounted:interfaces-state": {

"interface": [

{

"name": "ANI",

"bbf-interfaces-performance-management-mounted:performance": {

"intervals-15min": {

"history": [

{

"interval-number": "1",

"bbf-xpon-performance-management-mounted:xpon": {

"phy": {

"fec-seconds": 0,

"corrected-fec-codewords": "0",

"corrected-fec-bytes": "0",

"in-bip-errors": "0",

"in-fec-codewords": "4294967295",

"uncorrectable-fec-codewords": "0",

"in-bip-protected-words": "0"

}

}

}

]

}

}

}

]

}

}

}

]

}

}

}

]

}

}

##### CPU/MEM Counters

The CPU/MEM related counters are for NT and LT cards.

Select ipfix cache profile/LS-MF/cpu-mem-utilization

##### Disk utilization Counters

The CPU/MEM related counters are for NT and LT cards.

Select ipfix cache profile/LS-MF/disk-utilization

### PM collection configuration on AAC

PM bulk collection model parameter requires to configure on AAC

Under AAC/System administration/extensions/pm utilities

Bulk Collection destination, kafka & openTSDB, are defined.

Kafka topic, IPFIX-XPON, are also defined.

### Bulk collection NBI

Use kafka to get the Bulk PM counters from northbound OSS. IPFIX-XPON is the Kafka topic to be subscribed from OSS side.

Sample IPFIX result

| PON port gem port related counters | { "anv:device-manager": { "anv-device-holders:device": [ { "device-id": "SWOLT000044.LT1", "timestamp": "2023-07-13T09:40:00+0000", "device-specific-data": { "ietf-interfaces:interfaces-state": { "interface": [ { "name": "CT_SWOLT000044.L1P1_1_XGS", "statistics": { "nokia-sdan-xpon-statistics:xpon": { "out-incidental-broadcast-gem-port-bytes": "0", "out-incidental-broadcast-gem-port-dropped-packets": 0, "out-unicast-gem-port-dropped-packets": 0, "out-multicast-gem-port-dropped-packets": 0, "out-incidental-broadcast-gem-port-packets": "0", "out-multicast-gem-port-bytes": "0", "out-multicast-gem-port-dropped-bytes": "0", "out-unicast-gem-port-packets": "0", "out-multicast-gem-port-packets": "0", "out-incidental-broadcast-gem-port-dropped-bytes": "0", "out-unicast-gem-port-bytes": "0", "out-unicast-gem-port-dropped-bytes": "0" } } }, { "name": "CT_SWOLT000044.L1P1_1_GPON", "statistics": { "nokia-sdan-xpon-statistics:xpon": { "out-incidental-broadcast-gem-port-bytes": "0", "out-incidental-broadcast-gem-port-dropped-packets": 0, "out-unicast-gem-port-dropped-packets": 0, "out-multicast-gem-port-dropped-packets": 0, "out-incidental-broadcast-gem-port-packets": "0", "out-multicast-gem-port-bytes": "0", "out-multicast-gem-port-dropped-bytes": "0", "out-unicast-gem-port-packets": "0", "out-multicast-gem-port-packets": "0", "out-incidental-broadcast-gem-port-dropped-bytes": "0", "out-unicast-gem-port-bytes": "0", "out-unicast-gem-port-dropped-bytes": "0" } } } ] } } } ] } } |
| --- | --- |
| Uplink port counters | { "anv:device-manager": { "anv-device-holders:device": [ { "device-id": "SWOLT000044.IHUB", "timestamp": "2023-07-13T09:30:00+0000", "device-specific-data": { "nokia-state:state": { "port": [ { "port-id": "1/1/1", "ethernet": { "performance": { "intervals-15min": { "history": [ { "interval-number": "1", "out-pkts": "0", "in-octets": "0", "in-bandwidth": "0", "out-octets": "0", "in-pkts": "0", "out-bandwidth": "0" } ] } } } }, { "port-id": "1/2/1", "ethernet": { "performance": { "intervals-15min": { "history": [ { "interval-number": "1", "out-pkts": "1077", "in-octets": "1131880", "in-bandwidth": "0", "out-octets": "501833", "in-pkts": "1941", "out-bandwidth": "0" } ] } } } }, { "port-id": "1/3/1", "ethernet": { "performance": { "intervals-15min": { "history": [ { "interval-number": "1", "out-pkts": "0", "in-octets": "0", "in-bandwidth": "0", "out-octets": "0", "in-pkts": "0", "out-bandwidth": "0" } ] } } } }, { "port-id": "1/4/1", "ethernet": { "performance": { "intervals-15min": { "history": [ { "interval-number": "1", "out-pkts": "1077", "in-octets": "1131816", "in-bandwidth": "0", "out-octets": "501833", "in-pkts": "1940", "out-bandwidth": "0" } ] } } } }, { "port-id": "1/5/3", "ethernet": { "performance": { "intervals-15min": { "history": [ { "interval-number": "1", "out-pkts": "7113", "in-octets": "3017903", "in-bandwidth": "0", "out-octets": "2941824", "in-pkts": "8465", "out-bandwidth": "0" } ] } } } }, { "port-id": "1/6/c1", "ethernet": { "performance": { "intervals-15min": { "history": [ { "interval-number": "1", "out-pkts": "0", "in-octets": "0", "in-bandwidth": "0", "out-octets": "0", "in-pkts": "0", "out-bandwidth": "0" } ] } } } }, { "port-id": "1/6/c1/1", "ethernet": { "performance": { "intervals-15min": { "history": [ { "interval-number": "1", "out-pkts": "897", "in-octets": "0", "in-bandwidth": "0", "out-octets": "114816", "in-pkts": "0", "out-bandwidth": "0" } ] } } } } ] } } } ] } } |
| PON port in/out counters | { "anv:device-manager": { "anv-device-holders:device": [ { "device-id": "SWOLT000044.LT1", "timestamp": "2023-07-13T09:30:00+0000", "device-specific-data": { "ietf-interfaces:interfaces-state": { "interface": [ { "name": "CT_SWOLT000044.L1P1_1_XGS", "statistics": { "out-octets": "0", "out-discards": 0, "in-octets": "0", "in-discards": 0 } }, { "name": "CT_SWOLT000044.L1P1_1_GPON", "statistics": { "out-octets": "0", "out-discards": 0, "in-octets": "185506368", "in-discards": 4 } } ] } } } ] } } { "anv:device-manager": { "anv-device-holders:device": [ { "device-id": "SWOLT000044.LT1", "timestamp": "2023-07-13T09:30:00+0000", "device-specific-data": { "ietf-interfaces:interfaces-state": { "interface": [ { "name": "CT_SWOLT000044.L1P1_1_XGS", "statistics": { "out-octets": "0", "bbf-interfaces-statistics-management:in-pkts": "0", "nokia-sdan-additional-statistics:in-dropped-bytes": "0", "nokia-sdan-additional-statistics:out-dropped-bytes": "0", "out-discards": 0, "bbf-interfaces-statistics-management:out-pkts": "0", "in-octets": "0", "in-discards": 0 } }, { "name": "CT_SWOLT000044.L1P1_1_GPON", "statistics": { "out-octets": "0", "bbf-interfaces-statistics-management:in-pkts": "123342", "nokia-sdan-additional-statistics:in-dropped-bytes": "0", "nokia-sdan-additional-statistics:out-dropped-bytes": "0", "out-discards": 0, "bbf-interfaces-statistics-management:out-pkts": "0", "in-octets": "185506368", "in-discards": 4 } } ] } } } ] } } |
| BER related counters | Refer to chapter 4.8.3.2.7 |

## Inventory interface

There are two types of inventories,

- Intent inventory.

  - It is the device/OLT abstraction. It stored the intent YANG

  - We can classify the intent inventory with IBN type, such as device-mf, fiber, uplink, ont, l2-user…

  - Intent inventory data is stored in ES-db (Elastic Search DB, in later release it will be Open Search DB)

- Device inventory.

  - It is the device/OLT yang inventory.

  - It can be classified as hardware, interface, system, l2-forwarder, qos…

ES-db query is the OSS interface to query both IBN inventory and device inventory.

| Catalog | Function | METHOD | endpoint |
| --- | --- | --- | --- |
| Intent inventory Query | Query all Intent inventory | POST | altiplano-elasticsearch/intents/_search/ |
| Device inventory Query | Query all OLT inventory | POST | altiplano-elasticsearch/latestcompleted-inv/_search |

### ES-search in general

#### Filter for ElasticSearch

If we don’t specify the term to filter, ElasticSearch will dump all the data base. When the network become big, the response will taker longer time.

Better use Filter term in the ElasticSearch. It can help generate nice and neat result.

Sample intent inventory search as below:

GET {{protocol}}://{{server}}/altiplano-elasticsearch/intents/_search/

{

"query": {

"bool": {

"filter": {

"term": {

"intent-type": "fiber"

}

}

}

},

"_source": ["target.fiber-name","configuration"]

#### Retrieve selected fields form a ElasticSearch

By default, each hit in the search response includes the document _source, which is the entire JSON object that was provided when indexing the document.

Use the _source option if you need to access the original data that was passed at index time.

You can use both of these methods, though the fields option is preferred sample is as below

GET {{protocol}}://{{server}}/altiplano-elasticsearch/intents/_search/

{

"query": {

"bool": {

"filter": {

"term": {

"intent-type": "fiber"

}

}

}

},

"_source": ["target.fiber-name","configuration"]

#### Pagination for ElasticSearch

ElasticSearch is regarded as 3rd party APP. Refer to its Reference document: https://www.elastic.co/guide/en/elasticsearch/reference/7.x/paginate-search-results.html, “from” and “size” can be used for pagination. According to the Reference document, when hits result exceed 10000, use “search_after” instead.

### Intent inventory

Below are some intent inventory Elastic Search filter and field samples for reference. OSS can/should define its own filters and fields, based on the business logic.

#### query device-mf

“device-mf” refers to the mf2 OLT. Query device-mf can get the provisioned LT card list

GET {{protocol}}://{{server}}/altiplano-elasticsearch/intents/_search/

{

"query": {

"bool": {

"filter": {

"term": {

"intent-type": "device-mf"

}

}

}

},

"_source": ["target.device-name","nested-lists","configuration"]

}

Sample response are:

{

"took": 188,

"timed_out": false,

"_shards": {

"total": 75,

"successful": 75,

"skipped": 0,

"failed": 0

},

"hits": {

"total": {

"value": 2,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "intents-device-mf-v1",

"_type": "_doc",

"_id": "device-mf_$$_MF2_2012",

"_score": 0.0,

"_source": {

"configuration": {

"lt-device-template": [

"default"

],

"transport-protocol": [

"ssh"

],

"board-service-profile": [

"Embedded-ONT"

],

"slot-name": [

"LT1"

],

"ip-address": [

"135.251.214.235"

],

"password": [

"$-0$V3lf2dzPKZIczWEhDaQMezp/VqEu1gxEuK0DfnK7L2k="

],

"device-version": [

"20.12"

],

"device-template": [

"default"

],

"push-nav-configuration-to-device": [

"true"

],

"planned-type": [

"LWLT-C"

],

"hardware-type": [

"LS-MF-LMNT-A"

],

"ip-port": [

832

],

"device-manager": [

"Altiplano_AV"

],

"username": [

"admin"

]

},

"nested-lists": [

{

"lt-device-template": [

"default"

],

"board-service-profile": [

"Embedded-ONT"

],

"planned-type": [

"LWLT-C"

],

"slot-name": [

"LT1"

]

}

],

"target": {

"device-name": "MF2_2012"

}

}

},

{

"_index": "intents-device-mf-v1",

"_type": "_doc",

"_id": "device-mf_$$_C600_MF2",

"_score": 0.0,

"_source": {

"configuration": {

"lt-device-template": [

"default"

],

"transport-protocol": [

"ssh"

],

"board-service-profile": [

"Embedded-ONT"

],

"slot-name": [

"LT2"

],

"ip-address": [

"135.251.223.235"

],

"password": [

"$-0$eyzS2qBet74nA6rkCH0FgDVe3505xsEOhXQcViXvQFk="

],

"device-version": [

"21.3"

],

"device-template": [

"default"

],

"push-nav-configuration-to-device": [

"true"

],

"planned-type": [

"LWLT-C"

],

"hardware-type": [

"LS-MF-LMNT-A"

],

"ip-port": [

832

],

"device-manager": [

"Altiplano_AV"

],

"username": [

"admin"

]

},

"nested-lists": [

{

"lt-device-template": [

"default"

],

"planned-type": [

"LWLT-C"

],

"board-service-profile": [

"Embedded-ONT"

],

"slot-name": [

"LT2"

]

}

],

"target": {

"device-name": "C600_MF2"

}

}

}

]

}

}

#### query uplink-connection

uplink-connection intent refers to uplink port and LAG. Below Elastic Search can be used to get the uplink-connection related configuration.

GET {{protocol}}://{{server}}/altiplano-elasticsearch/intents/_search/

{

"query": {

"bool": {

"filter": {

"term": {

"intent-type": "uplink-connection"

}

}

}

},

"_source": ["target.device-name","configuration"]

}

Sample response are:

{

"took": 31,

"timed_out": false,

"_shards": {

"total": 60,

"successful": 60,

"skipped": 0,

"failed": 0

},

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "intents-uplink-connection-v5",

"_type": "_doc",

"_id": "uplink-connection_$$_SWOLT000044",

"_score": 0.0,

"_source": {

"configuration": {

"agg-system-name": [

"LAG-1-Dynamic-LACP-Active"

],

"connector-port-profile": [

"All-QSFP-4x10G-Breakout"

],

"port-id": [

"ltio-a-1:qsfp:1-1"

],

"l1-profile": [

"No-Negotiation-10Gbps"

]

},

"target": {

"device-name": "SWOLT000044"

}

}

}

]

}

}

#### query l2-infra

you can use below command to query the l2-infra, which link to the SVLAN service. L2-infra from MF-2 and FD-ISAM (in 23.12) can be retrieved by single cmd.

According to service module, l2-infra service refers to CVC, it can be created at the initial OLT provision.

GET {{protocol}}://{{server}}/altiplano-elasticsearch/intents/_search/

{

"query": {

"bool": {

"filter": {

"term": {

"intent-type": "l2-infra"

}

}

}

},

"_source": ["target.device-name","configuration","target.service-name"]

}

Sample response are:

{

"took": 36,

"timed_out": false,

"_shards": {

"total": 60,

"successful": 60,

"skipped": 0,

"failed": 0

},

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "intents-l2-infra-v7",

"_type": "_doc",

"_id": "l2-infra_$$_SWOLT000044_$_CVC21232140",

"_score": 0.0,

"_source": {

"configuration": {

"service-profile": [

"Unicast"

],

"vlan-mode": [

"cross-connect"

],

"forwarder-profile": [

"Unicast"

],

"service-description": [

"CVC103992"

],

"none": [

""

],

"s-vlan-id": [

5

],

"nni-id": [

"LAG-1"

]

},

"target": {

"service-name": "CVC21232140",

"device-name": "SWOLT000044"

}

}

}

]

}

}

#### query fiber

fiber is linking to the PON, below Elastic Search can be used to get the fiber list from both MF and FX OLT.

In the sample result, it shows 3 types of PON under different “fiber entity”

| OLT name | Fiber name | PON ID | PON type |
| --- | --- | --- | --- |
| SH-MF2 | SH-MF2.1.2 | LT1.PON2 | gpon |
| SH-MF2 | xgsFiber | LT1.PON1 | xgs |
| SH-MF2 | mpmFiber | LT1.PON4 | mpm-gpon-xgs |

GET {{protocol}}://{{server}}/altiplano-elasticsearch/intents/_search/

{

"query": {

"bool": {

"filter": {

"term": {

"intent-type": "fiber"

}

}

}

},

"_source": ["target.fiber-name","configuration"]

}

Sample response are:

{

"took": 3,

"timed_out": false,

"_shards": {

"total": 5,

"successful": 5,

"skipped": 0,

"failed": 0

},

"hits": {

"total": {

"value": 3,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "intents",

"_type": "_doc",

"_id": "fiber_$$_LT1.PON2",

"_score": 0.0,

"_source": {

"configuration": {

"xpon-type": [

"gpon"

],

"device-name": [

"SH-MF2"

],

"pon-id": [

"LT1.PON2"

],

"cage-mode": [

"auto"

],

"port-profile": [

"Default Port Profile"

],

"wavelength-profile": [

"auto"

],

"line-rate": [

"auto"

],

"model-name": [

"GPON SFP B+ (I-temp) OLT (3FE53441AA)"

]

},

"target": {

"fiber-name": "SH-MF2.1.2"

}

}

},

{

"_index": "intents",

"_type": "_doc",

"_id": "fiber_$$_xgsFiber",

"_score": 0.0,

"_source": {

"configuration": {

"xpon-type": [

"xgs"

],

"device-name": [

"SH-MF2"

],

"pon-id": [

"LT1.PON1"

],

"cage-mode": [

"auto"

],

"port-profile": [

"Default Port Profile"

],

"wavelength-profile": [

"auto"

],

"line-rate": [

"auto"

],

"model-name": [

"XG-PON1 XFP N1 (C-temp) OLT Asymmetric (3FE47548AA)"

]

},

"target": {

"fiber-name": "xgsFiber"

}

}

},

{

"_index": "intents",

"_type": "_doc",

"_id": "fiber_$$_mpmFiber",

"_score": 0.0,

"_source": {

"configuration": {

"xpon-type": [

"mpm-gpon-xgs"

],

"device-name": [

"SH-MF2"

],

"pon-id": [

"LT1.PON3"

],

"cage-mode": [

"auto"

],

"port-profile": [

"Default Port Profile"

],

"wavelength-profile": [

"auto"

],

"line-rate": [

"auto"

],

"model-name": [

"MPM B+(28dB) E-temp (3FE47581AF)"

]

},

"target": {

"fiber-name": "mpmFiber"

}

}

}

]

}

}

#### query ont

“ont” refers to ont intents. Query ont can get the provisioned ont list.

GET {{protocol}}://{{server}}/altiplano-elasticsearch/intents/_search/

{

"query": {

"bool": {

"filter": {

"term": {

"intent-type": "ont"

}

}

}

},

"_source": ["target.ont-name","configuration"]

}

Sample response are:

{

"took": 31,

"timed_out": false,

"_shards": {

"total": 60,

"successful": 60,

"skipped": 0,

"failed": 0

},

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "intents-ont-v6",

"_type": "_doc",

"_id": "ont_$$_NTD123456789023",

"_score": 0.0,

"_source": {

"configuration": {

"ont-type": [

"NTD-GPN-1GEx4-POTSx2-typA"

],

"service-profile": [

"UNIS-D100MTC4-LowTC2-TC1",

"UNIS-D1000MTC4"

],

"from-device-mapping": [

""

],

"onu-service-profile": [

"ONTS-DsShaper1GL1"

],

"pon-type": [

"gpon"

],

"expected-serial-number": [

"ALCLF81E1CAE"

],

"uni-id": [

"LAN4",

"LAN3",

"LAN2",

"LAN1"

],

"fiber-name": [

"SWOLT000044.L1P1"

],

"onu-id": [

1

]

},

"target": {

"ont-name": "NTD123456789023"

}

}

}

]

}

}

#### query l2-user

“l2-user” refers to AVC service. Query l2-user can get the provisioned AVC list.

GET {{protocol}}://{{server}}/altiplano-elasticsearch/intents/_search/

{

"query": {

"bool": {

"filter": {

"term": {

"intent-type": "l2-user"

}

}

}

},

"_source": ["target.device-name","configuration"]

}

Sample response are:

{

"took": 34,

"timed_out": false,

"_shards": {

"total": 60,

"successful": 60,

"skipped": 0,

"failed": 0

},

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "intents-l2-user-v9",

"_type": "_doc",

"_id": "l2-user_$$_NEBS_$_AVC12344662222",

"_score": 0.0,

"_source": {

"configuration": {

"max-number-mac-address": [

8

],

"l2-infra": [

"CVC21232140"

],

"service-profile": [

"NEBS_400MTC4-100MTC2-5MTC1"

],

"user-device-name": [

"NTD123456789023"

],

"speed-profile": [

"400MTC4-100MTC2-5MTC1"

],

"customer-id": [

"avc1232412313"

],

"q-vlan-id": [

100

],

"c-vlan-id": [

100

],

"uni-id": [

"LAN1"

]

}

}

}

]

}

}

### Device inventory

#### Importing the Inventory Collection Models

The Inventory Collection models for the NETCONF devices should be imported into the Inventory Manager Application before the inventory collection can be started. You can also import them using the “System Administration Application”.

Click on the Settings icon  and select Models. On the Models page, click on the import icon  and select the Collection models you want to import. The progress and then the result of the import action is displayed. The imported collection models are now available as cards in the Models page. It is recommended to import an inventory collection model when no existing inventory collection is in progress.

#### Schedule the collection and export

You can view the device level inventory collection details on the Collections page. The page has two tabs, NETCONF and SNMP, displaying the collection details for NETCONF and SNMP devices respectively.

On the Collections page, you can trigger the network inventory collection manually using the  button.  Only one active network-wide inventory collection type (NETCONF or SNMP) happens at any point in time, i.e. one NETCONF inventory collection can happen simultaneously with one SNMP inventory collection.

You can schedule the inventory collection to start automatically at a specified time. You can also set the inventory collection schedule to repeat after a specified interval, say, on a daily basis. An optional end time for the inventory collection can also be specified if you wish to stop triggering the inventory collection after that specified time.

#### Viewing the Inventory Collection Status

The result of the inventory collection is shown in the Collection Status table in the COLLECTION tab. Click on the Additional columns icon  on the top right-hand corner of the card to add or remove columns. Select or deselect the columns as per your needs and save the changes.

The status of a NETCONF inventory collection is given below.

You can view the collection mode (manually triggered or scheduled), the number of collection requests for the number of devices and the device mangers, successful collection and failed collection for the devices and the device managers, the username of the user who triggered the collection etc. The collection Status column displays the result of the inventory collection. The meaning of each status icon and description is given below.

| Collection Status Icon | Description | Remarks |
| --- | --- | --- |
| Collection Status Icon | Description | Remarks |
|  | Completed | The inventory collection is successfully completed |
|  | Completed | The inventory collection is completed, but it failed for one or more devices or device managers. |
|  | In progress | The inventory collection is in progress |

If the collection status is 'failed', check the additional information to find out the reason for the collection failure. By default, the first 100 results are displayed. Click on the 'Load More Device Info' button to display more information on the failure. You can filter the additional information to identify the causes of failure, for example, on specific devices, by typing the initial few characters of the device.

#### Customize inventory module

Please refer to below blue print device inventory module xml file.(take LT card xml as an example)

Highlight in yellow is not TRUE use case.

<?xml version="1.0" encoding="UTF-8"?>

<clientConfig xmlns="uri://nokia.com/inventory/collection/model/1.1"

>

<signature key=""/>

<!-- extra meta tag -->

<modelmeta modelName="fiber" modelVersion="1.0.0" description="Inventory collection model for fiber devices" family="LS-MF-LWLT-C-22.12" collectorType="NETCONF" author="NOKIA" date="26-Apr-2023 09:59" build="1.0.0-SNAPSHOT"/>

<collectedObjects esIndexMappingNestedObjectsLimit="250000">

<inventoryObject objectType="/bbf-l2-dhcpv4-relay:l2-dhcpv4-relay-profiles"                     includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-l2-forwarding:forwarding"                                     includeSubtrees="Y" collected="Y" depthLimit="7"/>

<inventoryObject objectType="/bbf-mgmd:multicast"                                               includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-pppoe-intermediate-agent:pppoe-profiles"                      includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-qos-classifiers:classifiers"                                  includeSubtrees="Y" collected="Y" depthLimit="6"/>

<inventoryObject objectType="/bbf-qos-filters:filters"                                          includeSubtrees="Y" collected="Y" depthLimit="6"/>

<inventoryObject objectType="/bbf-qos-policies:policies"                                        includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-qos-policing:policing-profiles"                               includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-qos-traffic-mngt:tm-profiles"                                 includeSubtrees="Y" collected="Y" depthLimit="6"/>

<inventoryObject objectType="/bbf-subscriber-profiles:subscriber-profiles"                      includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/ietf-hardware:hardware"                                           includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/ietf-interfaces:interfaces"                                       includeSubtrees="Y" collected="Y" depthLimit="6"/>

<inventoryObject objectType="/ietf-routing:routing"                                             includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/ietf-system:system"                                               includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/nokia-dhcp-client:dhcp-client"                                    includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-fiber-onu-emulated-mount:onus"                                includeSubtrees="Y" collected="Y" depthLimit="8"/>

<inventoryObject objectType="/bbf-fiber-onu-emulated-mount:cpu-load-control-cfg"                includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-link-table:link-table"                                        includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-xpon:wavelength-profiles"                                     includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-xpon:ictp-config"                                             includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-xpon:multicast-gemports-config"                               includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-xpon:multicast-distribution-set-config"                       includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-xpongemtcont:traffic-descriptor-profiles"                     includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-xpongemtcont:xpongemtcont/bbf-xpongemtcont:tconts"     includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/bbf-xpongemtcont:xpongemtcont/bbf-xpongemtcont:gemports"       includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/nokia-onu-extension:onu-number-name-map"                          includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/nokia-virtual-network-function:virtual-network-functions"         includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/nokia-hardware-protection-management:hardware-protection-group"   includeSubtrees="Y" collected="Y"/>

<inventoryObject objectType="/nokia-hardware-protection-management:hardware-protection-element" includeSubtrees="Y" collected="Y"/>

<!-- State Data -->

<inventoryObject objectType="/ietf-hardware:hardware-state/ietf-hardware:component/ietf-hardware:description"  collectStateData="Y" includeSubtrees="N" collected="Y"/>

<inventoryObject objectType="/ietf-hardware:hardware-state/ietf-hardware:component/ietf-hardware:hardware-rev"  collectStateData="Y" includeSubtrees="N" collected="Y"/>

<inventoryObject objectType="/ietf-hardware:hardware-state/ietf-hardware:component/ietf-hardware:serial-num"  collectStateData="Y" includeSubtrees="N" collected="Y"/>

<inventoryObject objectType="/ietf-hardware:hardware-state/ietf-hardware:component/ietf-hardware:mfg-name"  collectStateData="Y" includeSubtrees="N" collected="Y"/>

<inventoryObject objectType="/ietf-hardware:hardware-state/ietf-hardware:component/ietf-hardware:mfg-date"  collectStateData="Y" includeSubtrees="N" collected="Y"/>

<inventoryObject objectType="/ietf-hardware:hardware-state/ietf-hardware:component/ietf-hardware:model-name"  collectStateData="Y" includeSubtrees="N" collected="Y"/>

<inventoryObject objectType="/ietf-interfaces:interfaces-state/ietf-interfaces:interface/bbf-xponvani:v-ani/bbf-xponvani:onu-present-on-this-olt/bbf-xponvani:detected-serial-number"  collectStateData="Y" includeSubtrees="N" collected="Y"/>

<inventoryObject objectType="/ietf-interfaces:interfaces-state/ietf-interfaces:interface/bbf-xponvani:v-ani/bbf-xponvani:onu-present-on-this-olt/bbf-xponvani:onu-fiber-distance"  collectStateData="Y" includeSubtrees="N" collected="Y"/>

<inventoryObject objectType="/bbf-fiber-onu-emulated-mount:onus/bbf-fiber-onu-emulated-mount:onu/bbf-fiber-onu-emulated-mount:root/ietf-hardware-mounted:hardware-state/ietf-hardware-mounted:component/ietf-hardware-mounted:state/ietf-hardware-mounted:admin-state"  collectStateData="Y" includeSubtrees="N" collected="Y" depthLimit="8"/>

<inventoryObject objectType="/bbf-fiber-onu-emulated-mount:onus/bbf-fiber-onu-emulated-mount:onu/bbf-fiber-onu-emulated-mount:root/ietf-hardware-mounted:hardware-state/ietf-hardware-mounted:component/ietf-hardware-mounted:hardware-rev"  collectStateData="Y" includeSubtrees="N" collected="Y"/>

<inventoryObject objectType="/bbf-fiber-onu-emulated-mount:onus/bbf-fiber-onu-emulated-mount:onu/bbf-fiber-onu-emulated-mount:root/ietf-hardware-mounted:hardware-state/ietf-hardware-mounted:component/ietf-hardware-mounted:mfg-name"  collectStateData="Y" includeSubtrees="N" collected="Y"/>

<inventoryObject objectType="/bbf-fiber-onu-emulated-mount:onus/bbf-fiber-onu-emulated-mount:onu/bbf-fiber-onu-emulated-mount:root/ietf-hardware-mounted:hardware-state/ietf-hardware-mounted:component/ietf-hardware-mounted:model-name"  collectStateData="Y" includeSubtrees="N" collected="Y"/>

<inventoryObject objectType="/bbf-fiber-onu-emulated-mount:onus/bbf-fiber-onu-emulated-mount:onu/bbf-fiber-onu-emulated-mount:root/ietf-hardware-mounted:hardware-state/ietf-hardware-mounted:component/bbf-software-image-management-one-dot-one-mounted:software/bbf-software-image-management-one-dot-one-mounted:software/bbf-software-image-management-one-dot-one-mounted:revisions/bbf-software-image-management-one-dot-one-mounted:revision/bbf-software-image-management-one-dot-one-mounted:name"  collectStateData="Y" includeSubtrees="N" collected="Y" depthLimit="11"/>

<inventoryObject objectType="/bbf-fiber-onu-emulated-mount:onus/bbf-fiber-onu-emulated-mount:onu/bbf-fiber-onu-emulated-mount:root/ietf-hardware-mounted:hardware-state/ietf-hardware-mounted:component/bbf-software-image-management-one-dot-one-mounted:software/bbf-software-image-management-one-dot-one-mounted:software/bbf-software-image-management-one-dot-one-mounted:revisions/bbf-software-image-management-one-dot-one-mounted:revision/bbf-software-image-management-one-dot-one-mounted:is-active"  collectStateData="Y" includeSubtrees="N" collected="Y" depthLimit="11"/>

</collectedObjects>

</clientConfig>

#### Device inventory elastic search samples

Below are some device inventory Elastic Search filter and field samples for reference. OSS can/should define its own filters and fields, based on the business logic.

##### Query all

GET {{protocol}}:///{{server}}/altiplano-elasticsearch/latestcompleted-inv/_search

{

"_source": true,

"query": {

"match_all": {}

}

}

Sample response are:

##### Query CHASSIS

GET {{protocol}}:///{{server}}/altiplano-elasticsearch/latestcompleted-inv/_search

{

"_source": false,

"query": {

"nested": {

"path": "inventorydata.ietf-hardware:hardware.component",

"query": {

"bool": {

"filter": {

"term": {"inventorydata.ietf-hardware:hardware.component.class.keyword": {"value": "iana-hardware:chassis"}}

}

}

},"inner_hits": {"_source": ["inventorydata.ietf-hardware:hardware.component"]}

}

}

}

Sample response are:

{

"took": 66,

"timed_out": false,

"_shards": {

"total": 4,

"successful": 3,

"skipped": 0,

"failed": 1,

"failures": [

{

"shard": 0,

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]",

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"index_uuid": "fgL_wu8bTO6K4fZq6az3cQ",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]"

}

}

}

]

},

"hits": {

"total": {

"value": 3,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-hardware:hardware.component": {

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 0

}

},

"_score": 0.0,

"_source": {

"bbf-hardware-extension:model-name": "LMFS-A",

"mfg-name": "ALCL",

"name": "Chassis",

"admin-state": "unlocked",

"class": "iana-hardware:chassis"

}

}

]

}

}

}

},

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-hardware:hardware.component": {

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 0

}

},

"_score": 0.0,

"_source": {

"bbf-hardware-extension:model-name": "LMFS-A",

"mfg-name": "ALCL",

"name": "Chassis",

"admin-state": "unlocked",

"class": "iana-hardware:chassis"

}

}

]

}

}

}

},

{

"_index": "inv_sx4f-5.8.01_sx4f-std_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "sx4f-1",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-hardware:hardware.component": {

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_sx4f-5.8.01_sx4f-std_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "sx4f-1",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 1

}

},

"_score": 0.0,

"_source": {

"bbf-hardware-extension:model-name": "CFAS-J",

"mfg-name": "ALCL",

"name": "Chassis",

"admin-state": "unlocked",

"class": "iana-hardware:chassis"

}

}

]

}

}

}

}

]

}

}

##### Query slot-nt

GET {{protocol}}:///{{server}}/altiplano-elasticsearch/latestcompleted-inv/_search

{

"_source": false,

"query": {

"nested": {

"path": "inventorydata.ietf-hardware:hardware.component",

"query": {

"bool": {

"filter": {

"term": {"inventorydata.ietf-hardware:hardware.component.class.keyword": {"value": "nokia-hardware-identities:slot-nt"}}

}

}

},"inner_hits": {"_source": ["inventorydata.ietf-hardware:hardware.component"]}

}

}

}

Sample response are:

{

"took": 18,

"timed_out": false,

"_shards": {

"total": 4,

"successful": 3,

"skipped": 0,

"failed": 1,

"failures": [

{

"shard": 0,

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]",

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"index_uuid": "fgL_wu8bTO6K4fZq6az3cQ",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]"

}

}

}

]

},

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-hardware:hardware.component": {

"hits": {

"total": {

"value": 2,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 10

}

},

"_score": 0.0,

"_source": {

"parent": "Chassis",

"name": "Slot-Nt-A",

"admin-state": "unlocked",

"parent-rel-pos": 1,

"class": "nokia-hardware-identities:slot-nt"

}

},

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 11

}

},

"_score": 0.0,

"_source": {

"parent": "Chassis",

"name": "Slot-Nt-B",

"admin-state": "unlocked",

"parent-rel-pos": 2,

"class": "nokia-hardware-identities:slot-nt"

}

}

]

}

}

}

}

]

}

}

##### Query slot-lt

GET {{protocol}}:///{{server}}/altiplano-elasticsearch/latestcompleted-inv/_search

{

"_source": false,

"query": {

"nested": {

"path": "inventorydata.ietf-hardware:hardware.component",

"query": {

"bool": {

"filter": {

"term": {"inventorydata.ietf-hardware:hardware.component.class.keyword": {"value": "nokia-hardware-identities:slot-lt"}}

}

}

},"inner_hits": {"_source": ["inventorydata.ietf-hardware:hardware.component"]}

}

}

}

Sample response are:

{

"took": 40,

"timed_out": false,

"_shards": {

"total": 4,

"successful": 3,

"skipped": 0,

"failed": 1,

"failures": [

{

"shard": 0,

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]",

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"index_uuid": "fgL_wu8bTO6K4fZq6az3cQ",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]"

}

}

}

]

},

"hits": {

"total": {

"value": 2,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-hardware:hardware.component": {

"hits": {

"total": {

"value": 14,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 12

}

},

"_score": 0.0,

"_source": {

"parent": "Chassis",

"name": "Slot-Lt-1",

"admin-state": "unlocked",

"parent-rel-pos": 1,

"class": "nokia-hardware-identities:slot-lt"

}

},

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 13

}

},

"_score": 0.0,

"_source": {

"parent": "Chassis",

"name": "Slot-Lt-2",

"admin-state": "unlocked",

"parent-rel-pos": 2,

"class": "nokia-hardware-identities:slot-lt"

}

},

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 14

}

},

"_score": 0.0,

"_source": {

"parent": "Chassis",

"name": "Slot-Lt-3",

"admin-state": "unlocked",

"parent-rel-pos": 3,

"class": "nokia-hardware-identities:slot-lt"

}

}

]

}

}

}

},

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-hardware:hardware.component": {

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 1

}

},

"_score": 0.0,

"_source": {

"parent": "Chassis",

"name": "Slot",

"admin-state": "unlocked",

"parent-rel-pos": 1,

"class": "nokia-hardware-identities:slot-lt"

}

}

]

}

}

}

}

]

}

}

##### Query board-nt

GET {{protocol}}:///{{server}}/altiplano-elasticsearch/latestcompleted-inv/_search

{

"_source": false,

"query": {

"nested": {

"path": "inventorydata.ietf-hardware:hardware.component",

"query": {

"bool": {

"filter": {

"term": {"inventorydata.ietf-hardware:hardware.component.class.keyword": {"value": "nokia-hardware-identities:nt"}}

}

}

},"inner_hits": {"_source": ["inventorydata.ietf-hardware:hardware.component"]}

}

}

}

Sample response are:

{

"took": 40,

"timed_out": false,

"_shards": {

"total": 4,

"successful": 3,

"skipped": 0,

"failed": 1,

"failures": [

{

"shard": 0,

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]",

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"index_uuid": "fgL_wu8bTO6K4fZq6az3cQ",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]"

}

}

}

]

},

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-hardware:hardware.component": {

"hits": {

"total": {

"value": 2,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 26

}

},

"_score": 0.0,

"_source": {

"parent": "Slot-Nt-A",

"bbf-hardware-extension:model-name": "LANT-A",

"mfg-name": "ALCL",

"name": "Board-Nta",

"admin-state": "unlocked",

"parent-rel-pos": 1,

"class": "nokia-hardware-identities:nt"

}

},

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 27

}

},

"_score": 0.0,

"_source": {

"parent": "Slot-Nt-B",

"bbf-hardware-extension:model-name": "LANT-A",

"mfg-name": "ALCL",

"name": "Board-Ntb",

"admin-state": "unlocked",

"parent-rel-pos": 1,

"class": "nokia-hardware-identities:nt"

}

}

]

}

}

}

}

]

}

}

##### Query board-lt

GET {{protocol}}:///{{server}}/altiplano-elasticsearch/latestcompleted-inv/_search

{

"_source": false,

"query": {

"nested": {

"path": "inventorydata.ietf-hardware:hardware.component",

"query": {

"bool": {

"filter": {

"term": {"inventorydata.ietf-hardware:hardware.component.class.keyword": {"value": "nokia-hardware-identities:lt"}}

}

}

},"inner_hits": {"_source": ["inventorydata.ietf-hardware:hardware.component"]}

}

}

}

Sample response are:

{

"took": 45,

"timed_out": false,

"_shards": {

"total": 4,

"successful": 3,

"skipped": 0,

"failed": 1,

"failures": [

{

"shard": 0,

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]",

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"index_uuid": "fgL_wu8bTO6K4fZq6az3cQ",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]"

}

}

}

]

},

"hits": {

"total": {

"value": 2,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-hardware:hardware.component": {

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 28

}

},

"_score": 0.0,

"_source": {

"parent": "Slot-Lt-1",

"bbf-hardware-extension:model-name": "LWLT-C",

"mfg-name": "ALCL",

"name": "Board-LT1",

"parent-rel-pos": 1,

"class": "nokia-hardware-identities:lt"

}

}

]

}

}

}

},

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-hardware:hardware.component": {

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 2

}

},

"_score": 0.0,

"_source": {

"parent": "Slot",

"bbf-hardware-extension:model-name": "LWLT-C",

"mfg-name": "ALCL",

"name": "Board",

"admin-state": "unlocked",

"parent-rel-pos": 1,

"class": "nokia-hardware-identities:lt"

}

}

]

}

}

}

}

]

}

}

##### Query cage

GET {{protocol}}:///{{server}}/altiplano-elasticsearch/latestcompleted-inv/_search

{

"_source": false,

"query": {

"nested": {

"path": "inventorydata.ietf-hardware:hardware.component",

"query": {

"bool": {

"filter": {

"term": {"inventorydata.ietf-hardware:hardware.component.class.keyword": {"value": "bbf-hardware-types:cage"}}

}

}

},"inner_hits": {"_source": ["inventorydata.ietf-hardware:hardware.component"]}

}

}

}

Sample response are:

{

"took": 75,

"timed_out": false,

"_shards": {

"total": 4,

"successful": 3,

"skipped": 0,

"failed": 1,

"failures": [

{

"shard": 0,

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]",

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"index_uuid": "fgL_wu8bTO6K4fZq6az3cQ",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]"

}

}

}

]

},

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-hardware:hardware.component": {

"hits": {

"total": {

"value": 16,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 3

}

},

"_score": 0.0,

"_source": {

"parent": "Board",

"name": "C1",

"admin-state": "unlocked",

"parent-rel-pos": 1,

"class": "bbf-hardware-types:cage"

}

},

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 4

}

},

"_score": 0.0,

"_source": {

"parent": "Board",

"name": "C2",

"admin-state": "unlocked",

"parent-rel-pos": 2,

"class": "bbf-hardware-types:cage"

}

},

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_05-07",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware",

"offset": 0,

"_nested": {

"field": "component",

"offset": 5

}

},

"_score": 0.0,

"_source": {

"parent": "Board",

"name": "C3",

"admin-state": "unlocked",

"parent-rel-pos": 3,

"class": "bbf-hardware-types:cage"

}

}

]

}

}

}

}

]

}

}

##### Query transceiver

GET {{protocol}}:///{{server}}/altiplano-elasticsearch/latestcompleted-inv/_search

{

"_source": false,

"query": {

"nested": {

"path": "inventorydata.ietf-hardware:hardware.component",

"query": {

"bool": {

"filter": {

"term": {"inventorydata.ietf-hardware:hardware.component.class.keyword": {"value": "bbf-hardware-types:transceiver"}}

}

}

},"inner_hits": {"_source": ["inventorydata.ietf-hardware:hardware.component"]}

}

}

}

Sample response are:

{

"took": 4,

"timed_out": false,

"_shards": {

"total": 4,

"successful": 3,

"skipped": 0,

"failed": 1,

"failures": [

{

"shard": 0,

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]",

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_05-07",

"index_uuid": "fgL_wu8bTO6K4fZq6az3cQ",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware.component]"

}

}

}

]

},

"hits": {

"total": {

"value": 0,

"relation": "eq"

},

"max_score": null,

"hits": []

}

}

##### Query transceiver-link

GET {{protocol}}:///{{server}}/altiplano-elasticsearch/latestcompleted-inv/_search

{

"_source": false,

"query": {

"nested": {

"path": "inventorydata.ietf-hardware:hardware-state.component.bbf-hardware-transceivers:transceiver-link",

"query": {

"bool": {

}

},"inner_hits": {"_source": ["inventorydata.ietf-hardware:hardware-state.component.bbf-hardware-transceivers:transceiver-link"]}

}

}

}

Sample response are:

{

"took": 74,

"timed_out": false,

"_shards": {

"total": 3,

"successful": 1,

"skipped": 0,

"failed": 2,

"failures": [

{

"shard": 0,

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-08-11_04-16",

"node": "aKpcCIwlQ7y729IwaNjuig",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware-state.component.bbf-hardware-transceivers:transceiver-link]",

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-08-11_04-16",

"index_uuid": "MRg_598PTZSpfQs3289mGg",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware-state.component.bbf-hardware-transceivers:transceiver-link]"

}

}

},

{

"shard": 0,

"index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-08-11_04-16",

"node": "aKpcCIwlQ7y729IwaNjuig",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware-state.component.bbf-hardware-transceivers:transceiver-link]",

"index": "inv_ls-mf-lant-a-22.12_ls-mf-lant_1.0.0_2023-08-11_04-16",

"index_uuid": "nqPuPeVKRTODhSXIHSaljg",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-hardware:hardware-state.component.bbf-hardware-transceivers:transceiver-link]"

}

}

}

]

},

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 1.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-08-11_04-16",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_score": 1.0,

"inner_hits": {

"inventorydata.ietf-hardware:hardware-state.component.bbf-hardware-transceivers:transceiver-link": {

"hits": {

"total": {

"value": 2,

"relation": "eq"

},

"max_score": 1.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-08-11_04-16",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware-state",

"offset": 0,

"_nested": {

"field": "component",

"offset": 28,

"_nested": {

"field": "bbf-hardware-transceivers:transceiver-link",

"offset": 0

}

}

},

"_score": 1.0,

"_source": {

"wavelength": 1490

}

},

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-08-11_04-16",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-hardware:hardware-state",

"offset": 0,

"_nested": {

"field": "component",

"offset": 29,

"_nested": {

"field": "bbf-hardware-transceivers:transceiver-link",

"offset": 0

}

}

},

"_score": 1.0,

"_source": {

"wavelength": 1577

}

}

]

}

}

}

}

]

}

}

##### Query channel-pair

GET {{protocol}}:///{{server}}/altiplano-elasticsearch/latestcompleted-inv/_search

{

"_source": false,

"query": {

"nested": {

"path": "inventorydata.ietf-interfaces:interfaces.interface",

"query": {

"bool": {

"filter": {

"term": {"inventorydata.ietf-interfaces:interfaces.interface.type.keyword": {"value": "bbf-xpon-if-type:channel-pair"}}

}

}

},"inner_hits": {"_source": ["inventorydata.ietf-interfaces:interfaces.interface"]}

}

}

}

Sample response are:

{

"took": 63,

"timed_out": false,

"_shards": {

"total": 6,

"successful": 3,

"skipped": 0,

"failed": 3,

"failures": [

{

"shard": 0,

"index": "inv_504vg-1.0__2023-07-15_11-14",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]",

"index": "inv_504vg-1.0__2023-07-15_11-14",

"index_uuid": "APdRSyF0SGmtG0BbgJh3rw",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]"

}

}

},

{

"shard": 0,

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_11-14",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]",

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_11-14",

"index_uuid": "EeiPaGNBTA238yxN8PiavA",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]"

}

}

},

{

"shard": 0,

"index": "inv_ndd-4100-1.5__2023-07-15_11-14",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]",

"index": "inv_ndd-4100-1.5__2023-07-15_11-14",

"index_uuid": "e5v44eTfRa-bS1rPfcapBw",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]"

}

}

}

]

},

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_11-14",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-interfaces:interfaces.interface": {

"hits": {

"total": {

"value": 2,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_11-14",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-interfaces:interfaces",

"offset": 0,

"_nested": {

"field": "interface",

"offset": 7

}

},

"_score": 0.0,

"_source": {

"bbf-xpon:channel-pair": {

"bbf-xpon-ploam-authentication:onu-ploam-authentication-fail-control": "bbf-xpon-ploam-authentication:keep-onu-ranged",

"nokia-sdan-if-xpon-aug:ber-tca-profile": "init-ber-tca-default",

"channel-partition-ref": "SWOLT000044.L1P1_CPART_GPON",

"channel-pair-type": "bbf-xpon-types:gpon",

"gpon-pon-id-interval": 0,

"channel-group-ref": "SWOLT000044.L1P1"

},

"nokia-interfaces-statistics:statistics": {

"enable": "best-effort"

},

"name": "SWOLT000044.L1P1_CPAIR_GPON",

"nokia-multicast-cac:multicast-cac": {

"max-group-number": "no-limit",

"max-multicast-rate-limit": "no-limit",

"multicast-rate-limit-exceed-action": "best-effort"

},

"type": "bbf-xpon-if-type:channel-pair",

"enabled": true,

"bbf-interfaces-performance-management:performance": {

"enable": false

}

}

},

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_11-14",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-interfaces:interfaces",

"offset": 0,

"_nested": {

"field": "interface",

"offset": 10

}

},

"_score": 0.0,

"_source": {

"bbf-xpon:channel-pair": {

"bbf-xpon-ploam-authentication:onu-ploam-authentication-fail-control": "bbf-xpon-ploam-authentication:keep-onu-ranged",

"nokia-sdan-if-xpon-aug:ber-tca-profile": "init-ber-tca-default",

"channel-partition-ref": "SWOLT000044.L1P1_CPART_XGS",

"channel-pair-type": "bbf-xpon-types:xgs",

"wavelength-prof-ref": "xgspon-wave",

"channel-group-ref": "SWOLT000044.L1P1"

},

"nokia-interfaces-statistics:statistics": {

"enable": "best-effort"

},

"name": "SWOLT000044.L1P1_CPAIR_XGS",

"nokia-multicast-cac:multicast-cac": {

"max-group-number": "no-limit",

"max-multicast-rate-limit": "no-limit",

"multicast-rate-limit-exceed-action": "best-effort"

},

"type": "bbf-xpon-if-type:channel-pair",

"enabled": true,

"bbf-interfaces-performance-management:performance": {

"enable": false

}

}

}

]

}

}

}

}

]

}

}

##### Query channel-partition

GET {{protocol}}:///{{server}}/altiplano-elasticsearch/latestcompleted-inv/_search

{

"_source": false,

"query": {

"nested": {

"path": "inventorydata.ietf-interfaces:interfaces.interface",

"query": {

"bool": {

"filter": {

"term": {"inventorydata.ietf-interfaces:interfaces.interface.type.keyword": {"value": "bbf-xpon-if-type:channel-partition"}}

}

}

},"inner_hits": {"_source": ["inventorydata.ietf-interfaces:interfaces.interface"]}

}

}

}

Sample response are:

{

"took": 65,

"timed_out": false,

"_shards": {

"total": 6,

"successful": 3,

"skipped": 0,

"failed": 3,

"failures": [

{

"shard": 0,

"index": "inv_504vg-1.0__2023-07-15_11-14",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]",

"index": "inv_504vg-1.0__2023-07-15_11-14",

"index_uuid": "APdRSyF0SGmtG0BbgJh3rw",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]"

}

}

},

{

"shard": 0,

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_11-14",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]",

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_11-14",

"index_uuid": "EeiPaGNBTA238yxN8PiavA",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]"

}

}

},

{

"shard": 0,

"index": "inv_ndd-4100-1.5__2023-07-15_11-14",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]",

"index": "inv_ndd-4100-1.5__2023-07-15_11-14",

"index_uuid": "e5v44eTfRa-bS1rPfcapBw",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]"

}

}

}

]

},

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_11-14",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-interfaces:interfaces.interface": {

"hits": {

"total": {

"value": 2,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_11-14",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-interfaces:interfaces",

"offset": 0,

"_nested": {

"field": "interface",

"offset": 6

}

},

"_score": 0.0,

"_source": {

"bbf-qos-traffic-mngt:tm-root": {

"tc-id-2-queue-id-mapping-profile-name": "DEFAULT_TC_TO_8Queues",

"bbf-qos-enhanced-scheduling:scheduler-node": [

{

"name": "NTD123456789023",

"shaper-name": "sp984.5M",

"scheduling-level": 1,

"child-scheduler-nodes": [

{

"name": "NTD123456789023_LAN1",

"weight": 0

},

{

"name": "NTD123456789023_LAN2",

"weight": 0

},

{

"name": "NTD123456789023_LAN3",

"weight": 0

},

{

"name": "NTD123456789023_LAN4",

"weight": 0

}

]

},

{

"contains-queues": true,

"name": "NTD123456789023_LAN1",

"scheduling-level": 2,

"nokia-qos-queue-monitoring-extension:queue-monitoring": {

"enable-statistics": false,

"enable-performance": false

},

"queue": [

{

"bac-name": "UNI_DS_TC1_Q",

"weight": 0,

"local-queue-id": 1,

"priority": 7

},

{

"bac-name": "UNI_DS_TC2_Q_Low",

"weight": 0,

"local-queue-id": 2,

"priority": 6

},

{

"bac-name": "illegal",

"weight": 66,

"local-queue-id": 3,

"priority": 5

},

{

"bac-name": "UNI_DS_TC4_Q",

"weight": 34,

"local-queue-id": 4,

"priority": 5

}

]

},

{

"contains-queues": true,

"name": "NTD123456789023_LAN2",

"scheduling-level": 2,

"nokia-qos-queue-monitoring-extension:queue-monitoring": {

"enable-statistics": false,

"enable-performance": false

},

"queue": [

{

"bac-name": "UNI_DS_TC1_Q",

"weight": 0,

"local-queue-id": 1,

"priority": 7

},

{

"bac-name": "UNI_DS_TC2_Q_Low",

"weight": 0,

"local-queue-id": 2,

"priority": 6

},

{

"bac-name": "illegal",

"weight": 66,

"local-queue-id": 3,

"priority": 5

},

{

"bac-name": "UNI_DS_TC4_Q",

"weight": 34,

"local-queue-id": 4,

"priority": 5

}

]

},

{

"contains-queues": true,

"name": "NTD123456789023_LAN3",

"scheduling-level": 2,

"nokia-qos-queue-monitoring-extension:queue-monitoring": {

"enable-statistics": false,

"enable-performance": false

},

"queue": [

{

"bac-name": "UNI_DS_TC1_Q",

"weight": 0,

"local-queue-id": 1,

"priority": 7

},

{

"bac-name": "UNI_DS_TC2_Q_Low",

"weight": 0,

"local-queue-id": 2,

"priority": 6

},

{

"bac-name": "illegal",

"weight": 66,

"local-queue-id": 3,

"priority": 5

},

{

"bac-name": "UNI_DS_TC4_Q",

"weight": 34,

"local-queue-id": 4,

"priority": 5

}

]

},

{

"contains-queues": true,

"name": "NTD123456789023_LAN4",

"scheduling-level": 2,

"nokia-qos-queue-monitoring-extension:queue-monitoring": {

"enable-statistics": false,

"enable-performance": false

},

"queue": [

{

"bac-name": "illegal",

"weight": 0,

"local-queue-id": 1,

"priority": 7

},

{

"bac-name": "illegal",

"weight": 0,

"local-queue-id": 2,

"priority": 6

},

{

"bac-name": "illegal",

"weight": 66,

"local-queue-id": 3,

"priority": 5

},

{

"bac-name": "UNI_DS_TC4_Q",

"weight": 34,

"local-queue-id": 4,

"priority": 5

}

]

}

],

"bbf-qos-enhanced-scheduling:child-scheduler-nodes": [

{

"name": "NTD123456789023",

"weight": 50,

"priority": 0

}

]

},

"nokia-interfaces-statistics:statistics": {

"enable": "best-effort"

},

"bbf-xpon:channel-partition": {

"closest-onu-distance": 0,

"authentication-method": "per-v-ani-expected",

"channel-partition-index": 1,

"downstream-fec": true,

"maximum-differential-xpon-distance": 20,

"multicast-aes-indicator": false,

"channel-group-ref": "SWOLT000044.L1P1"

},

"name": "SWOLT000044.L1P1_CPART_GPON",

"type": "bbf-xpon-if-type:channel-partition",

"enabled": true,

"bbf-interfaces-performance-management:performance": {

"enable": false

}

}

},

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_11-14",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-interfaces:interfaces",

"offset": 0,

"_nested": {

"field": "interface",

"offset": 9

}

},

"_score": 0.0,

"_source": {

"bbf-qos-traffic-mngt:tm-root": {

"tc-id-2-queue-id-mapping-profile-name": "DEFAULT_TC_TO_8Queues"

},

"nokia-interfaces-statistics:statistics": {

"enable": "best-effort"

},

"bbf-xpon:channel-partition": {

"closest-onu-distance": 0,

"authentication-method": "per-v-ani-expected",

"channel-partition-index": 2,

"downstream-fec": true,

"maximum-differential-xpon-distance": 20,

"multicast-aes-indicator": false,

"channel-group-ref": "SWOLT000044.L1P1"

},

"name": "SWOLT000044.L1P1_CPART_XGS",

"type": "bbf-xpon-if-type:channel-partition",

"enabled": true,

"bbf-interfaces-performance-management:performance": {

"enable": false

}

}

}

]

}

}

}

}

]

}

}

##### Query channel-terminal

GET {{protocol}}:///{{server}}/altiplano-elasticsearch/latestcompleted-inv/_search

{

"_source": false,

"query": {

"nested": {

"path": "inventorydata.ietf-interfaces:interfaces.interface",

"query": {

"bool": {

"filter": {

"term": {"inventorydata.ietf-interfaces:interfaces.interface.type.keyword": {"value": "bbf-xpon-if-type:channel-termination"}}

}

}

},"inner_hits": {"_source": ["inventorydata.ietf-interfaces:interfaces.interface"]}

}

}

}

Sample response are:

{

"took": 95,

"timed_out": false,

"_shards": {

"total": 6,

"successful": 3,

"skipped": 0,

"failed": 3,

"failures": [

{

"shard": 0,

"index": "inv_504vg-1.0__2023-07-15_11-14",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]",

"index": "inv_504vg-1.0__2023-07-15_11-14",

"index_uuid": "APdRSyF0SGmtG0BbgJh3rw",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]"

}

}

},

{

"shard": 0,

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_11-14",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]",

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_11-14",

"index_uuid": "EeiPaGNBTA238yxN8PiavA",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]"

}

}

},

{

"shard": 0,

"index": "inv_ndd-4100-1.5__2023-07-15_11-14",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]",

"index": "inv_ndd-4100-1.5__2023-07-15_11-14",

"index_uuid": "e5v44eTfRa-bS1rPfcapBw",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]"

}

}

}

]

},

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_11-14",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-interfaces:interfaces.interface": {

"hits": {

"total": {

"value": 2,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_11-14",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-interfaces:interfaces",

"offset": 0,

"_nested": {

"field": "interface",

"offset": 8

}

},

"_score": 0.0,

"_source": {

"bbf-xpon:channel-termination": {

"channel-pair-ref": "SWOLT000044.L1P1_CPAIR_GPON",

"gpon-pon-id": "00000000000000",

"nokia-sdan-if-xpon-aug:laser-on-by-default": true,

"location": "bbf-xpon-types:inside-olt",

"ber-calc-period": 10,

"channel-termination-type": "bbf-xpon-types:gpon"

},

"nokia-interfaces-statistics:statistics": {

"enable": "best-effort"

},

"name": "CT_SWOLT000044.L1P1_1_GPON",

"bbf-interface-port-reference:port-layer-if": [

"PON_1_GPON"

],

"type": "bbf-xpon-if-type:channel-termination",

"enabled": true,

"bbf-interfaces-performance-management:performance": {

"enable": false

}

}

},

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_11-14",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-interfaces:interfaces",

"offset": 0,

"_nested": {

"field": "interface",

"offset": 11

}

},

"_score": 0.0,

"_source": {

"bbf-xpon:channel-termination": {

"channel-pair-ref": "SWOLT000044.L1P1_CPAIR_XGS",

"nokia-sdan-if-xpon-aug:laser-on-by-default": true,

"location": "bbf-xpon-types:inside-olt",

"ber-calc-period": 10,

"channel-termination-type": "bbf-xpon-types:xgs"

},

"nokia-interfaces-statistics:statistics": {

"enable": "best-effort"

},

"name": "CT_SWOLT000044.L1P1_1_XGS",

"bbf-interface-port-reference:port-layer-if": [

"PON_1_XGS"

],

"type": "bbf-xpon-if-type:channel-termination",

"enabled": true,

"bbf-interfaces-performance-management:performance": {

"enable": false

}

}

}

]

}

}

}

}

]

}

}

##### Query ont

GET {{protocol}}:///{{server}}/altiplano-elasticsearch/latestcompleted-inv/_search

{

"_source": false,

"query": {

"nested": {

"path": "inventorydata.ietf-interfaces:interfaces.interface",

"query": {

"bool": {

"filter": {

"term": {"inventorydata.ietf-interfaces:interfaces.interface.type.keyword": {"value": "bbf-xpon-if-type:v-ani"}}

}

}

},"inner_hits": {"_source": ["inventorydata.ietf-interfaces:interfaces.interface"], "size":100}

}

}

}

Sample response are:

{

"took": 52,

"timed_out": false,

"_shards": {

"total": 6,

"successful": 3,

"skipped": 0,

"failed": 3,

"failures": [

{

"shard": 0,

"index": "inv_504vg-1.0__2023-07-15_11-14",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]",

"index": "inv_504vg-1.0__2023-07-15_11-14",

"index_uuid": "APdRSyF0SGmtG0BbgJh3rw",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]"

}

}

},

{

"shard": 0,

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_11-14",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]",

"index": "inv_ls-mf-ihub-lant-a-22.12_fiber-ihub_1.0.0_2023-07-15_11-14",

"index_uuid": "EeiPaGNBTA238yxN8PiavA",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]"

}

}

},

{

"shard": 0,

"index": "inv_ndd-4100-1.5__2023-07-15_11-14",

"node": "GwAgoB-3StapMjIyPTPZDw",

"reason": {

"type": "query_shard_exception",

"reason": "failed to create query: [nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]",

"index": "inv_ndd-4100-1.5__2023-07-15_11-14",

"index_uuid": "e5v44eTfRa-bS1rPfcapBw",

"caused_by": {

"type": "illegal_state_exception",

"reason": "[nested] failed to find nested object under path [inventorydata.ietf-interfaces:interfaces.interface]"

}

}

}

]

},

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_11-14",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_score": 0.0,

"inner_hits": {

"inventorydata.ietf-interfaces:interfaces.interface": {

"hits": {

"total": {

"value": 1,

"relation": "eq"

},

"max_score": 0.0,

"hits": [

{

"_index": "inv_ls-mf-lwlt-c-22.12_fiber_1.0.0_2023-07-15_11-14",

"_type": "_doc",

"_id": "SWOLT000044.LT1",

"_nested": {

"field": "inventorydata.ietf-interfaces:interfaces",

"offset": 0,

"_nested": {

"field": "interface",

"offset": 16

}

},

"_score": 0.0,

"_source": {

"nokia-interfaces-statistics:statistics": {

"enable": "best-effort"

},

"bbf-xponvani:v-ani": {

"channel-partition": "SWOLT000044.L1P1_CPART_GPON",

"nokia-conf-clock-tod-ctrl:tod-enable": false,

"upstream-fec": true,

"preferred-channel-pair": "SWOLT000044.L1P1_CPAIR_GPON",

"expected-serial-number": "ALCLF81E1CAE",

"management-gemport-aes-indicator": false,

"nokia-sdan-if-xponvani-aug:onu-name": "NTD123456789023",

"onu-id": 1

},

"name": "NTD123456789023",

"type": "bbf-xpon-if-type:v-ani",

"enabled": true,

"bbf-interfaces-performance-management:performance": {

"enable": false

}

}

}

]

}

}

}

}

]

}

}